1 					  LIST X

2 					; THIS IS THE MODIFIED SEPTEMBER ATARI 400/800 COMPUTER OPERATING

3 					; SYSTEM LISTING. MODIFIED TO ASSEMBLE ON THE MICROTEC CROSS

4 					; ASSEMBLER.

5 					; THIS VERSION IS THE ONE WHICH WAS BURNED INTO ROM.

6 		; THERE IS A RESIDUAL PIECE OF CODE WHICH IS FOR LNDUG. THIS

7 					; IS AT LOCATION $9000 WHICH IS NOT IN ROM.

8 					;

9 					; THIS IS THE REVISION D EPROM VERSION

10		   			;.PAGE

1l					;

12					;

13					; COLLEEN OPERATING	SYSTEM EQUATE FILE

14					;

15					; NTSC/PAL ASSEMBLY	FLAG

16					;

17	0000				PALFLG	= 	0	; 0 = NTSC 1 = PAL

18					;

19					;

20					;		MODULE ORIGIN TABLE

21					;

22	E000				CHRORG   	=	$E000		;CHARACTER SET

23	E400				VECTBL   	=	$E400		;VECTOR TAHLE

24	E480				VCTABL   	=	$E480		;RAM VECTOR INITIAL VALUE TABLE

25	E4A6				CIOORG   	=	$E4A6		;CENTRAL I/0 HANDLER

26	E6D5				INTORG   	=	$E6D5		;INTERRUPT HANDLER

27	E944				SIOORG   	=	$E944		;SERIAL I/0 DRIVER

28	EDEA				DSKORG   	=	$EDEA		;DISK HANDLER

29	EE78				PRNORG   	=	$EE78		;PRINTER HANDLER

30	EF41				CASORG   	=	$EF41		;CASSETTE HANDLER

31	F0E3				MONORG   	=	$F0E3		;MONITOR/POWER UP MODULE

32	F3E4				KBDORG   	=	$F3E4		;KEYBOARD/DISPLAY HANDLER

33					;

34					;

35					;

36					;

37					;	VECTOR TABLE

38					;

39					; HANDLER ENTRY POINTS ARE CALLED OUT IN THE FOLLOWING VECTOR

40					; TABLE. THESE ARE THE ADDRESSES MINUS ONE.

41					;

42					;

43					; EXAMPLE FOR EDITOR

44					;

45					;	E400 		OPEN

46					;  	   2 		CLOSE

47					;   	   4 		GET

48					;   	   6 		PUT

49					;   	   8 		STATUS

50					;   	   A 		SPECIAL

51					;   	   C 		JUMP TO POWER ON INITIALIZATION ROUTINE

52					;	   F 		NOT USED

53					;

54

55	E400				EDITRV   	=	$E400		;EDITOR

56	E410				SCRENV   	=	$E410		;TELEVISION SCREEN

57	E420				KEYBDV   	=	$E420		;KEYBOARD

58	E430				PRINTV   	=	$E430		;PRINTER

59	E440				CASETV   	=	$E440		;CASSETTE

60					;

6l					;		JUMP VECTOR TABLE

62					;

63					; THE FOLLOWING IS A TABLE	OF JUMP INSTRUCTIONS

64					; TO VARIOUS ENTRY POINTS IN	THE OPERATING SYSTEM.

65					;

66	E450				DISKIV   	=	$E450		;DISK INITIALIZATION

67	E453				DSKINV   	= 	$E453		;DISK INTERFACE

68	E456				CIOV     	=	$E456		;CENTRAL INPUT OUTPUT ROUTINE

69	E459				SIOV     	=	$E459		;SERIAL INPUT OUTPUT ROUTINE

70	E45C				SETVBV   	=	$E45C		;SET SYSTEM TIMERS ROUTINE

71	E45F				SYSVBV   	=	$E45F		;SYSTEM VERTICAL BLANK CALCULATIONS

72	E462				XITVBV   	=	$E462		;EXIT VERTICAL DLANK CALCULATIONS

73	E465				SIOINV   	=	$E465		;SERIAL INPUT OUTPUT INITIALIZATION

74	E468				SENDEV   	=	$E468		;SEND ENABLE ROUTINE

75	E46B				INTINV   	=	$E46B		;INTERRUPT HANDLER INITIALIZATION

76	E46E				CIOINV   	=	$E46E		;CENTRAL INPUT OUTPUT INITIALIZATION

77	E471				DLKDDV   	=	$E471		;BLACKBOARD MODE

78	E474				WARMSV   	=	$E474		;WARM START ENTRY POINT

79	E477				COLDSV   	=	$E477		;COLD START ENTRY POINT

80	E47A				RBLOKV   	=	$E47A		;CASSETTE READ BLOCK ENTRY POINT VECTOR

81	E47D				CSOPIV   	=	$E47D		;CASSETTE OPEN FOR INPUT VECTOR

82					;VCTABL  	=	$E480

83					;

84					;

85					;  	OPERATING SYSTEM EQUATES

86					;

87					; 	COMMAND CODES FOR IOCB

BB	0003				OPEN     	=	3		;OPEN FOR INPUT/OUTPUT

89	0005				GETREC   	=	5		;GET RECORD (TEXT)

90	0007				GETCHR   	=	7		;GET CHARACTERS)

91	0009				PUTREC   	=	9		;PUT RECORD (TEXT)

92	000B				PUTCHR   	=	$B		;PUT CHARACTERS)

93	000C				CLOSE    	=	$C		;CLOSE DEVICE

94	000D				STATIS   	=	$D		;STATUS REQUEST

95	000E				SPECIL   	=	$E		;BEGINNING OF SPECIAL ENTRY COMMANDS

96					;

97					; 		SPECIAL ENTRY COMMANDS

98	0011				DRAWLN   	=	$11		;DRAW LINE

99	0012				FILLIN   	=	$12		;DRAW LINE WITH RIGHT FILL

100	0020				RENAME   	=	$20		;RENAME DISK FILE

101	0021				DELETE   	=	$21		;DELETE DISK FILE

102	0022				FORMAT   	=	$22		;FORMAT

103	0023				LOCKFL   	=	$23		;LOCK FILE TO READ ONLY

104	0024				UNLOCK   	=	$24		;UNLOCK LOCKED FILE

105	0025				POINT    	=	$25		;POINT SECTOR

106	0026				NOTE     	=	$26		;NOTE SECTOR

107	D0FF				IOCFRE   	=	$FF		;IOCB "FREE"

108					;

109					; 	AUX1 EQUATES

110					;	() INDICATES WHICH DEVICES USE BIT

111	0001				APPEND   	=	$1		;OPEN FOR WRITE APPEND (D), OR SCREEN READ

112	0002				DIRECT   	=	$2		;OPEN FOR DIRECTORY ACCESS (D)

113	0004				OPNIN    	=	$4		;OPEN FOR INPUT (ALL DEVICES)

114	000B				OPNOT    	=	$8		;OPEN FOR OUTPUT (ALL DEVICES)

115	000C				OPNINO   	=	OPNIN+OPNOT	;OPEN FOR INPUT AND OUTPUT (ALL DEVICES)

116	0010				MXDMOD   	=	$10		;OPEN FOR MIXED MODE (E, S)

117	0020				INSCLR   	=	$20		;OPEN WITHOUT CLEARING SCREEN (E, S)

118					;

119					; DEVICE NAMES

120	0045				SCREDT   	=	'E		;SCREEN EDITOR (R/W)

121	004B				KBD	   	=	'K.		;KEYBOARD (R ONLY)

122	0053				DISPLY   	=	'S		;SCREEN DISPLAY (R/W)

123	0050				PRINTR   	=	'P		;PRINTER (W ONLY)

124	0043				CASSET   	=	'C		;CASSETTE

125	004D				MODEM	   	=	'M		;MODEM

126	0044				DISK    	=	'D		;DISK (R/W)

127					;

128					; SYSTEM EOL (CARRIAGE RETURN)

129	009D				CR	  	=	$9B

130					;

131					;

132					; OPERATING SYSTEM STATUS CODES

133					;

134	0001				SUCCES  	=	$01		;SUCCESSFUL OPERATION

135					;

136	0080				BRKABT  	=	$80		;BREAK KEY ABORT

137	0081				PRVOPN  	=	$81		;IOCB ALREADY OPEN

138	0082				NONDEV  	=	$82		;NON-EXISTANT DEVICE

139	0083				WRONLY  	=	$83		;IOCD OPENED FOR WRITE ONLY

140	0084				NVALID  	=	$84		;INVALID COMMAND

141	0085				NOTOPN  	=	$85		;DEVICE OR FILE NOT OPEN

142	0086				BADIOC  	=	$86		;INVALID IOCB NUMBER

143	0087				RDONLY  	=	$87		;IOCB OPENED FOR READ ONLY

144	0088				EOFERR  	= 	$88		;END OF FILE

145	0089				TRNRCD  	= 	$89		;TRUNCATED RECORD

146	008A				TIMOUT  	=	$8A		;PERIPHERAL DEVICE TIME OUT

147	008B				DNACK	  	=	$8B		;DEVICE DOES NOT ACKNOWLEDGE COMMAND

148	008C				FRMERR  	=	$8C		;SERIAL BUS FRAMING ERROR

149	008D				CRSROR  	=	$8D		;CURSOR OVERRANGE

150	008E				OVRRUN  	=	$8E		;SERIAL DUS DATA OVERRUN

151	008F				CHKERR  	=	$8F		;SERIAL BUS CHECKSUM ERROR

152					;

153	0090				DERROR  	=	$90		;PERIPHERAL DEVICE ERROR (OPERATION NOT COMP

154	0091				BADMOD  	=	$91		;BAD SCREEN MODE NUMBER

155	0092				FNCNOT  	=    	$92		;FUNCTION NOT IMPLEMENTED IN HANDLER

156	0093				SCRMEM  	=    	$93		;INSUFICIENT MEMORY FOR SCREEN MODE

157					;

158					;

159					;

160					;

l6l					;

162					;

163					;	PAGE ZERO RAM	ASSIGNMENTS

164					;

165							*=$0000

166	0000				LINZBS:	.RES 	2		;LINBUG RAM (WILL DE REPLACED BY MONITOR RAM

167					;

168					; THESE LOCATIONS ARE NOT CLEARED

169	0002				CASINI:	.RES 	2		;CASSETTE INIT LOCATION

170	0004				RAMLO:	.RES 	2		;RAM POINTER FOR MEMORY TEST

171	0006				TRAMSZ:	.RES 	1		;TEMPORARY REGISTER FOR RAM SIZE

172	0007				TSTDAT:	.RES 	1		;RAM TEST DATA REGISTER

173					;

174					; CLEARED ON COLDSTART ONLY

175	0008				WARMST:	.RES 	1		;WARM START FLAG

176	0009				B00T?:	.RES 	1		;SUCCESSFUL B00T FLAG

177	000A				DOSVEC:	.RES 	2		;DISK SOFTWARE START VECTOR

178	000C				DOSINI:	.RES 	2		;DISK SOFTWARE INIT ADDRESS

179	000E				APPMHI:	.RES 	2		;APPLICATIONS MEMORY HI LIMIT

180					;

181					; CLEAR ED ON COLD OR WARM START

182	0010				INTZBS   	=*				;INTERRUPT HANDLER

183	0010				POKMSK:	.RES 	1		;SYSTEM MASK FOR POKEY IRa ENABLE

184	0011				BRKKEY:	.RES 	1		;BREAK KEY FLAG

185	0012				RTCLOK:	.RES 	3		;REAL TIME CLOCK (IN 16 MSEC UNITS)

186					;

187	0015				BUFADR:	.RES 	2		;INDIRECT BUFFER ADDRESS REGISTER

188					;

189	0017				ICCOMT:	.RES 	1		;COMMAND FOR VECTOR

190					;

191	0018				DSKFMS:	.RES 	2		;DISK FILE MANAGER POINTER

192	001A				DSKUTL:	.RES 	2		;DISK UTILITIES POINTER

193

194	001C				PTIMOT:	.RES 	1		;PRINTER TIME OUT REGISTER

195	001D				PBPNT:	.RES 	1		;PRINT BUFFER POINTER

196	001E				PBUFSZ:	.RES 	1		;PRINT DUFFER SIZE

197	001F				PTEMP:	.RES 	1		;TEMPORARY REGISTER

198					;

199	0020				ZIOCB	   	=*			;ZERO PAGE I/0 CONTROL DLOCK

200	0010				IOCBSZ   	=     16		;NUMBER OF BYTES PER IOCB

201	0080				MAXIOC   	=     8*IOCBSZ	;LENGTH OF THE IOCB AREA

202	0020				IOCBAS   	=*

203	0020				ICHIDZ:	.RES 	1		;HANDLER INDEX NUMBER (FF = IOCB FREE)

204	0021				ICDNOZ:	.RES 	1		;DEVICE NUMBER (DRIVE NUMBER)

205	0022				ICCOMZ:	.RES 	1		;COMMAND CODE

206	0023				ICSTAZ:	.RES 	1		;STATUS OF LAST IOCB ACTION

207	0024				ICBALZ:	.RES 	1		;BUFFER ADDRESS LOW BYTE

208	0025				ICBAHZ:	.RES 	1

209	0026				ICPTLZ:	.RES 	1		;PUT BYTE ROUTINE ADDRESS - 1

210	0027				ICPTHZ:	.RES 	1

211	0028				ICBLLZ:	.RES 	1		;BUFFER LENGTH LOW BYTE

212	0029				ICBLHZ:	.RES 	1

213	002A				ICAXIZ:	.RES 	1		;AUXILIARY INFORMATION FIRST BYTE

214	002B				ICAX21:	.RES 	1

215	002C				ICSPRZ:	.RES 	4		;TWO SPARE BYTES (CIO LOCAL USE)

216	002E				ICIDNO   	=     ICSPRZ+2	;IOCB NUMBER X 16

217	002E				CIOCHR   	=     ICSPRZ+3	;CHARACTER BYTE FOR CURRENT OPERATION

218					;

219	0030				STATUS:	.RES 	1		;INTERNAL STATUS STORAGE

220	0031				CHKSUM:	.RES 	1		;CHECKSUM (SINGLE BYTE SUM WITH CARRY)

221	0032				BUFRLO:	.RES 	1		;POINTER TO DATA BUFFER (LO BYTE)

222	0033				BUFRHI:	.RES 	1		;POINTER TO DATA BUFFER (HI BYTE)

223	0034				BFENLO:	.RES 	1		;NEXT BYTE PAST END OF THE DATA DUFFER (LO BYTE)

224	0035				BFENHI:	.RES 	1		;NEXT BYTE PAST END OF THE DATA BUFFER (HI BYTE)

225	0036				CRETRY:	.RES 	1		;NUMBER OF COMMAND FRAME RETRIES

226	0037				DRETRY:	.RES 	1		;NUMBER OF DEVICE RETRIES

227	0038				BUFRFL:	.RES 	1		;DATA BUFFER FULL FLAG

228	0039				RECVDN:	.RES 	1		;RECEIVE DONE FLAG

229	003A				XMTDON:	.RES 	1		;TRANSMISSION DONE FLAG

230	003E				CHKSNT:	.RES 	1		;CHECKSUM SENT FLAG

231	003C				NOCKSM:	.RES 	1		;NO CHECKSUM FOLLOWS DATA FLAG

232					;

233					;

234	003D				BPTR:		.RES 	1

235	003E				FTYPE:	.RES 	1

236	003F				FE0F:		.RES 	1

237	0040				FREQ:		.RES 	1

238	0041				SOUNDR:	.RES 	1		;NOISY I/O FLAG. (ZERO IS QUIET)

239	0042				CRITIC:	.RES 	1		;DEFINES CRITICAL SECTION (CRITICAL IF NON-Z)

240					;

241	0043				FMSZPG:	.RES 	7		;DISK FILE MANAGER SYSTEM ZERO PAGE

242					;

243					;

244	004A				CKEY:		.RES 	1		;FLAG SET WHEN GAME START PRESSED

245	004E				CASSBT:	.RES 	1		;CASSETTE B00T FLAG

246	004C				DSTAT:	.RES 	1		;DISPLAY STATUS

247					;

248	004D				ATRACT:	.RES 	1		;ATRACT FLAG

249	004E				DRKMSK:	.RES 	1		;DARK ATRACT MASK

250	004E				COLRSH:	.RES 	1		;ATRACT COLOR SHIFTER (EOR'ED WITH PLAYFIELD

251					;

252	0002				LEDGE    	=    	2		;LMARGN'S VALUE AT COLD START

253	0027				REDGE	   	=    	39		;RMARGN'S VALUE AT COLD START

254	0050				TMPCHR:	.RES 	1

255	0051				HOLD1:	.RES 	1

256	0052				LMARGN:	.RES 	1		;LEFT MARGIN ISET TO i AT POWER ON>

257	0053				RMARGN:	.RES 	1		;RIGHT MARGIN (SET TO 38 AT POWER ON)

258	0054				ROWCRS:	.RES 	1		;CURSOR COUNTERS

259	0055				COLCRS:	.RES 	2

260	0057				DINDEX:	.RES 	1

2bi	0058				SAVMSC:	.RES 	2

262	005A				OLDROW:	.RES 	1

263	005B				OLDCOL:	.RES 	2

264	005D				OLDCHR:	.RES 	1		;DATA UNDER CURSOR

265	005E				OLDADR:	.RES 	2

266	0060				NEWROW:	.RES 	1		;POINT DRAW GOES TO

267	0061				NEWCOL:	.RES 	2

268	0063				LOGCOL:	.RES 	1		;POINTS AT COLUMN IN LOGICAL LINE

269	0064				ADRESS:	.RES 	2

270	0066				MLTTMP:	.RES 	2

271	0066				DPNTMP   	=     MLTTMP		;FIRST BYTE IS USED IN OPEN AS TEMP

272	0068				SAVADR:	.RES 	2

273	006A				RAMTOP:	.RES 	1		;RAM SIZE DEFINED BY POWER ON LOGIC

274	006B				BUFCNT:	.RES 	1		;BUFFER COUNT

275	006C				BUFSTR:	.RES 	2		;EDITOR GETCH POINTER

276	006E				BITMSK:	.RES 	1		;BIT MASK

277	006F				SHFAMT:	.RES 	1

278	0070				ROWAC:	.RES 	2

279	0072				COLAC:	.RES 	2

280	0074				ENDPT:	.RES 	2

281	0076				DELTAR:	.RES 	1

282	0077				DELTAC:	.RES 	2

283	0079				ROWINC:	.RES 	1

284	007A				COLINC:	.RES 	1

285	007E				SWPFLG:	.RES 	1	;NON-O IF TXT AND REGULAR RAM IS SWAPPED

286	007C				HOLDCH:	.RES 	1	;CH IS MOVED HERE IN KGETCH BEFORE CNTL & SH

287	007D				INSDAT:	.RES 	1

288	007E				COUNTR:	.RES 	2

289					;

290					;

291					;

292					;

293					;		80 -	FF ARE RESERVED FOR USER APPLICATIONS

294					;

295					;

296					;

297					;		NOTE	: SEE FLOATING POINT SUBROUTINE AREA FOR ZERO PAGE CELLS

298					;

299					;

300					;

301					;

302					;		PAGE 1 - STACK

303					;

304					;

305					;

306					;

307					;		PAGE TWO RAM ASSIGNMENTS

308					;

309							*=$0200

310	0200				INTABS	=*		;INTERRUPT RAM

311	0200				VDSLST:	.RES 	2	;DISPLAY LIST NMI VECTOR

312	0202				VPRCED:	.RES 	2	;PROCEED LINE IRQ VECTOR

313	0204				VINTER:	.RES 	2	;INTERRUPT LINE IRQ VECTOR

314	0206				VBREAK:	.RES 	2	;SOFTWARE BREAK (00) INSTRUCTION IRQ VECTOR

315	0208				VKEYBD:	.RES 	2	;POKEY KEYBOARD IRQ VECTOR

316	020A				VSERIN:	.RES 	2	;POKEY SERIAL INPUT READY IRQ

317	020C				VSEROR:	.RES 	2	;POKEY SERIAL OUTPUT READY IRQ

318	020E				VSEROC:	.RES 	2	;POKEY SERIAL OUTPUT COMPLETE IRQ

319	0210				VTIMR1:	.RES 	2	;POKEY TIMER 1 IRQ

320	0212				VTIMR2:	.RES	2	;POKEY TIMER 2 IRQ

321	0214				VTIMR4:	.RES	2	;POKEY TIMER 4 IRQ

322	0216				VIMIRQ:	.RES	2	;IMMEDIATE IRQ VECTOR

323	0218				CDTMV1:	.RES	2	;COUNT DOWN TIMER 1

324	021A				CDTMV2:	.RES	2	;COUNT DOWN TIMER 2

325	021C				CDTMV3:	.RES	2	;COUNT DOWN TIMER 3

326	021E				CDTMV4:	.RES	2	;COUNT DOWN TIMER 4

327	0220				CDTMV5:	.RES	2	;COUNT DOWN TIMER 5

328	0222				VVBLKI:	.RES	2	;IMMEDIATE VERTICAL BLANK NMI VECTOR

329	0224				VVBLKD:	.RES	2	;DEFERRED VERTICAL BLANK NMI VECTOR

330	0226				CDTMA1:	.RES	2	;COUNT DOWN TIMER 1 JSR ADDRESS

331	0228				CDTMA2:	.RES	2	;COUNT DOWN TIMER 2 JSR ADDRESS

332	022A				CDTMF3:	.RES	1	;COUNT DOWN TIMER 3 FLAG

333	022E				SRTIMR:	.RES	1	;SOFTWARE REPEAT TIMER

334	022C				CDTMF4:	.RES	1	;COUNT DOWN TIMER 4 FLAG

335	022D				INTEMP:	.RES	1	;IAN'S TEMP (RENAMED FROM T1 BY POPULAR DEMA)

336	022E				CDTMF5:	.RES	1	;COUNT DOWN TIMER FLAG 5

337	022E				SDMCTL:	.RES	1	;SAVE DMACTL REGISTER

338	0230				SDLSTL:	.RES	1	;SAVE DISPLAY LIST LOW BYTE

339	0231				SDLSTH:	.RES	1	;SAVE DISPLAY LIST HI BYTE

340	0232				SSKCTL:	.RES	1	;SKCTL REGISTER RAM

341	0233						.RES	1	;

342					;

343	0234				LPENH:	.RES	1	;LIGHT PEN HORIZONTAL VALUE

344	0235				LPENV:	.RES	1	:LIGHT PEN VERTICAL VALUE

345	0236				BRKKY:	.RES	2	;BREAK KEY VECTOR

346					;

347	0238						.RES	2	;SPARE

348					;

349	023A				CDEVIC:	.RES	1	;COMMAND FRAME BUFFER - DEVICE

350	023E				CCOMND:	.RES	1	;COMMAND

351	023C				CAUX1:	.RES	1	;COMMAND AUX BYTE 1

352	023D				CAUX2:	.RES	1	;COMMAND AUX BYTE 2

353					; NOTE: MAY	NOT BE THE LAST WORD ON A PAGE

354	023E				TEMP:		.RES	1	;TEMPORARY RAM CELL

355					; NOTE: MAY	NOT BE THE LAST WORD ON A PAGE

356	023E				ERRFLG:	.RES	1	;ERROR FLAG - ANY DEVICE ERROR EXCEPT TIME

357					;

358	0240				DFLAGS:	.RES	1	;DISK FLAGS FROM SECTOR ONE

359	0241				DBSECT:	.RES	1	;NUMBER OF DISK B00T SECTORS

360	0242				B00TAD:	.RES	2	;ADDRESS WHERE DISK B00T LOADER	WILL BE PUT

361	0244				COLDST:	.RES	1	;COLDSTART FLAG (1=IN MIDDLE OF	COLDSTART)

362					;

363	0245						.RES	1	;SPARE

364					;

365	0246				DSKTIM:	.RES	1	;DISK TIME OUT REGISTER

366					;

367	0247				LINBUF:	.RES	40	;CHAR LINE BUFFER

368					;

369	026E				GPRIOR:	.RES	1	;GLOBAL PRIORITY CELL

370					;

371	0270				PADDL0:	.RES	1	;POTENTIOMETER 0 RAM CELL

372	0271				PADDL1:	.RES	1

373	0272				PADDL2:	.RES	1

374	0273				PADDL3:	.RES	1

375	0274				PADDL4:	.RES	1

376	0275				PADDL5:	.RES	1

377	0276				PADDL6:	.RES	1

378	0277				PADDL7:	.RES	1

379	0278				STICK0:	.RES	1	;JOYSTICK 0 RAM CELL

380	0279				STICK1:	.RES	1

381	027A				STICK2:	.RES	1

382	0278				STICK3:	.RES	1

383	027C				PTRIGO:	.RES	1	;PADDLE TRIGGER 0

384	027D				PTRIGI:	.RES	1

385	027E				PTRI02:	RES	1

386	027E				PTRIG3:	.RES	1

387	0280				PTRIG4:	.RES	1

388	0281				PTRIG5:	.RES	1

389	0282				PTRIG6:	.RES	1

390	0283				PTRIG7:	.RES	1

391	0284				STRIG0:	.RES	1	;JOYSTICK TRIGGER 0

392	0285				STRIG1:	.RES	1

393	0286				STRIG2:	.RES	1

394	0287				STRIG3:	.RES	1

395					;

396	0288				CSTAT:	.RES	1

397	0289				WMODE:	.RES	1

398	028A				SLIM:	.RES	1

399	028E				IMASK:	.RES	1

400	028C				JVECK:	.RES	2

401					;

402	028E						.RES	2	;SPARE

403					;

404					;

405					;

406					;

407	0290				TXTROW:	.RES	1	;TEXT ROWCRS

408	0291				TXTCOL:	.RES	2	;TEXT COLCRS

409	0293				TINDEX:	.RES	1	;TEXT INDEX

410	0294				TXTMSC:	.RES	2	;F00LS CONVRT INTO NEW MSC

411	0296				TXTOLD:	.RES	6	;OLDROW & OLDCOL FOR TEXT (AND THEN SOME)

412	029C				TMPX1:	.RES	1

413	029D				HOLD3:	.RES	1

414	029E				SUBTMP:	.RES	1

415	029E				HOL02:	.RES	1

416	02A0				DMASK:	.RES	1

417	02A1				TMPLBT:	.RES	1

418	02A2				ESCFLG:	.RES	1	;ESCAPE FLAG

419	02A3				TABMAP:	.RES	15

420	0282				LOGMAP:	.RES	4	;LOGICAL LINE START BIT MAP

421	0286				INVFLG:	.RES	1	;INVERSE VIDEO FLAG (TOGGLED BY ATARI KEY)

422	0287				FILFLG:	.RES	1	;RIGHT FILL FLAG FOR DRAW

423	0288				TMPROW:	.RES	1

424	0289				TMPCOL:	.RES	2

425	02BB				SCRFLG:	.RES	1	;SET IF SCROLL OCCURS

426	02BC				HOLD4:	.RES	1	;TEMP CELL USED IN DRAW ONLY

427	02BD				HOLD5:	.RES	1	;DITTO

428	02BE				SHFLOK:	.RES	1

429	02BF				BOTSCR:	.RES	1	;BOTTOM OF SCREEN : 24 NORM 4 SPLIT

430					;

431					;

432	02C0				PCOLR0:	.RES	1	;P0 COLOR

433	02C1				PCOLR1:	.RES	1	;P1 COLOR

434	02C2				PCOLR2:	.RES	1	;P2 COLOR

435	02C3				PCOLR3:	.RES	1	;P3 COLOR

436	02C4				COLOR0:	.RES	1	;COLOR 0

437	02C5				COLOR1:	.RES	1

438	02C6				COLOR2:	.RES	1

439	02C7				COLOR3:	.RES	1

440	02CB				COLOR4:	.RES	1

441					;

442					;

443	02C9						.RES	23	;SPARE

444					;

445					;

446					;

447	02E0				GLBABS	=*		;GLOBAL VARIABLES

448					;

449	02E0						.RES	4	;SPARE

450					;

451	02E4				RAMSIZ:	.RES	1	;RAM SIZE (HI BYTE ONLY)

452	02E5				MEMTOP:	.RES	2	;TOP OF AVAILABLE USER MEMORY

453	02E7				MEMO:		.RES	2	;BOTTOM OF AVAILABLE USER MEMORY

454	02E9						.RES	1	;SPARE

455	02EA				DVSTAT:	.RES	4	;STATUS BUFFER

456	02EE				CBAUDL:	.RES	1	;CASSETTE BAUD RATE LOW BYTE

457	02EF				CBAUDH:	.RES	1

458					;

459	02F0				CRSINH:	.RES	1	;CURSOR INHIBIT (00 = CURSOR ON)

460	02F1				KEYDEL:	.RES	1	;KEY DELAY

461	02F2				CH1:		.RES	1

462

463	02F3				CHACT:	.RES	1	;CHACTL REGISTER RAM

464	02F4				CHBAS:	.RES	1	;CHBAS REGISTER RAM

465

466	02F5						.RES	5	;SPARE BYTES

467

468	02FA				CHAR:		.RES	1

469	02FB				ATACHR:	.RES	1	;ATASCII CHARACTER

470	02FC				CH:		.RES	1		;GLOBAL VARIABLE FOR KEYBOARD

471	02FD				FILDAT:	.RES	1	;RIGHT FILL DATA (DRAW)

472	02FE				DSPFLG:	.RES	1	;DISPLAY FLAG : DISPLAY CNTLS IF NON-ZERO

473	02FF				SSFLAG:	.RES	1	;START/STOP FLAG FOR PAGING (CNTL 1). CLEARE

474					;

475					;

476					;

477					;

478					;

479					;

480					;

481					;	PAGE	THREE RAM	ASSIGNMENTS

482					;

483	0300				DCB	=*			;DEVICE CONTROL BLOCK

484	0300				DDEVIC:	.RES	1	;PERIPHERAL UNIT 1 BUS I. D. NUMBER

485	0301				DUNIT:	.RES	1	;UNIT NUMBER

486	0302				DCOMND:	.RES	1	:BUS COMMAND

487	0303				DSTATS:	.RES	1	;COMMAND TYPE/STATUS RETURN

488	0304				DBUFLO:	.RES	1	;DATA BUFFER POINTER LOW BYTE

489	0305				DBUFHI:	.RES	1

490	0306				DTIMLO:	.RES	i	;DEVICE TIME OUT IN 1 SECOND UNITS

491	0307				DUNUSE:	.RES	1	;UNUSED BYTE

492	0308				DBYTLO:	.RES	1	;NUMBER OF BYTES TO BE TRANSFERRED LOW BYTE

493	0309				DBYTHI:	.RES	1

494	030A				DAUX1:	.RES	1	:COMMAND AUXILIARY BYTE 1

495	030E				DAUX2:	.RES	1

496					;

497	030C				TIMERI:	.RES	2	;INITIAL TIMER VALUE

498	030E				ADDCOR:	.RES	1	;ADDITION CORRECTION

499	030E				CASFLG:	.RES	1	;CASSETTE MODE WHEN SET

500	0310				TIMER2:	.RES	2	;FINAL TIMER VALUE. THESE TWO TIMER VALUES

501					; ARE USED TO COMPUTE INTERVAL FOR BAUD RATE

502	0312				TEMP1:	.RES	2	;TEMPORARY STORAGE REGISTER

503	0314				TEMP2:	.RES	1	;TEMPORARY STORAGE REGISTER

504	0315				TEMP3:	.RES	1	;TEMPORARY STORAGE REGISTER

505	0316				SAVIO:	.RES	1	;SAVE SERIAL IN DATA PORT

506	0317				TIMFLG:	.RES	1	;TIME OUT FLAG FOR BAUD RATE CORRECTION

507	0318				STACKP:	.RES	1	;SIO STACK POINTER SAVE CELL

508	0319				TSTAT:	.RES	1	:TEMPORARY STATUS HOLDER

509					;

510					;

511					;

512	031A				HATABS:	.RES	38	;HANDLER ADDRESS TABLE

513	0021				MAXDEV	=*-HATABS-5	;MAXIMUM HANDLER ADDRESS INDEX

514

515					;	NOTE :	THE ENTIRE IOCB DEFINITIONS HAVE BEEN MODIFIED

516					;

517					IOCB:		.ORG	*	;I/O CONTROL BLOCKS

518	0340				ICHID:	.RES	1	;HANDLER INDEX NUMBER (FF = IOCB FREE)

519	0341				ICDNO:	.RES	1	;DEVICE NUMBER (DRIVE NUMBER)

520	0342				ICCOM:	.RES	1	;COMMAND CODE

521	0343				ICSTA:	.RES	1	;STATUS OF LAST IOCB ACTION

522	0344				ICBAL:	.RES	1	;BUFFER ADDRESS LOW BYTE

523	0345				ICBAH:	.RES	1

524	0346				ICPTL:	.RES	1	;PUT BYTE ROUTINE ADDRESS - 1

525	0347				ICPTH:	.RES	1

526	0348				ICBLL:	.RES	1	;BUFFER LENGTH LOW BYTE

527	0349				ICBLH:	.RES	1

528	034A				ICAX1:	.RES	1	;AUXILIARY INFORMATION FIRST BYTE

529	034E				ICAX2:	.RES	1

530	034C				ICSPR:	.RES	4	;FOUR SPARE BYTES

531	0350						.RES	MAXIOC-IOCBSZ

532					;

533	03C0				PRNBUF:	.RES	40	:PRINTER BUFFER

534					;

535	03E8						.RES	21	;SPARE BYTES

536					;

537					;

538					;

539					;

540					;

541					;

542					;

543					;	PAGE	FOUR RAM ASSIGNMENTS

544					;

545	03FD				CASBUF:	.RES	131	;CASSETTE BUFFER

546					;

547					; USER	AREA STARTS HERE AND	GOES TO END OF PAGE FIVE

548	0480				USAREA:	.RES	128	;SPARE

549					;

550					;

551					;

552					;

553					;

554					;

555					;

556					;	PAGE FIVE RAM ASSIGNMENTS

557					;

558					;	PAGE FIVE IS RESERVED AS A USER WORK SPACE

559					;

560					;	NOTE: SEE FLOATING POINT SUBROUTINE AREA FOR PAGE FIVE CELLS

561					;

562					;

563					;	PAGE SIX RAM ASSIGNMENTS

564					;

565					; PAGE	SIX IS RESERVED	AS A USER'S USER WORK SPACE

566					;

567					;

568					;

569					;

570					;	FLOATING POINT	SUBROUTINES

571					;

572	0006				FPREC	   	=	6		;FLOATING POINT PRECISION (# OF BYTES)

573					; IF CARRY USED THEN CARRY CLEAR -> NO ERROR, CARR

574	D800				AFP	   	=	$D800		;ASCII->FLOATING POINT (FP)

575					;		INBUFF+CIX -> FR0, CIX, CARRY

576	DBE6				FASC	   	=	$D8E6		;FP -> ASCII FR0 -> LBUFF (INBUFF)

577	D9AA				IFP	   	=	$D9AA		;INTEGER -> FP

578					;		0-$FFFF (LSB, MSB) IN FR0, FR0+1 -> FR0

579	D9D2				FPI	   	=	$D9D2		;FP -> INTEGER FR0 -> FR0, FR0+1, CARRY

580	DA60				FSUB	   	=	$DA60		;FR0 <- FR0 – FR1 CARRY

581	DA66				FADD	   	=	$DA66		;FR0 <- FR0 + FR1 CARRY

582	DADS				FMUL	   	=	$DADB		;FR0 <- FR0 * FR1 CARRY

583	DB28				FDIV	   	=	$DB28		;FR0 <- FR0 / FR1 .CARRY

584	DD89				FLD0R	   	=	$DD89		;FLOATING LOAD REG0 FR0 <- (X, Y)

585	DD8D				FLD0P	   	=	$DD8D		; 	"	"  REG0 FR0 <- (FLPTR)

586	DD98				FLD1R	   	=	$DD98		; 	"	"  REG1 FR1 <- (X, Y)

587	DD9C				FLDlP	   	=	$DD9C		; 	"	"  REG1 FR1 <- (FLPTR)

588	DDA7				FST0R	   	=	$DDA7		;FLOATING STORE REG0 (X, Y) <- FR0

589	DDAB				FST0P    	=	$DDAB		; 	" 	"   REG0 (FLPTR) <- FR0

590	DDB6				FMOVE	   	=	$DDB6		;FR1 <- FR0

591	DD40				PLYEVL   	= 	$DD40		;FR0 <- P(Z) = SUM(I=N TO 0) (A(I)*Z**I) CAR

592					;		INPUT: (X, Y) = A(N), A(N-1). . . A(O) -> PLYARG

593					;		ACC = # 0F COEFFICIENTS = DEGREE+1

594					;		FR0 = Z

595	DDC0				EXP	   	=	$DDCO		;FR0 <- E**FR0 = EXP10(FR0 * LOG10(E)) CARRY

596	DDCC				EXP10	   	=	$DDCC		;FR0 <- 10**FR0 CARRY

597	DECD				LOG	   	=	$DECD		;FR0 <- LN(FR0) = LOG10(FR0)/LOG10(E) CARRY

598	DED1				LOG10	   	=	$DED1		;FR0 <- LOG10(FR0) CARRY

599					; 	THE FOLLOWING ARE IN	BASIC CARTRIDGE:

600	BD81				SIN	   	=	$BD81		;FR0 <- SIN(FR0) DEGFLG=0 =>RADS, 6=>DEG. CA

601	BD73				COS	   	=	$BD73		;FR0 <- COS(FR0) CARRY

602	BE43				ATAN	   	=	SBE43		;FR0 <- ATAN(FR0) CARRY

603	BEB1				SQR	   	= 	$BEB1		;FR0 <- SQUARER00T(FR0) CARRY

604					; FLOATING POINT ROUTINES ZERO PAGE (NEEDED ONLY IF F. P. ROUTINES ARE CA

605								*=$D4

606	00D4				FR0:		.RES	FPREC		;FP REG0

607	00DA				FRE:		.RES	FPREC

608	00E0				FRI:		.RES	FPREC		;FP REG1

609	00E6				FR2:		.RES	FPREC

610	00EC				FRX:		.RES	1		;FP SPARE

611	00ED				EEXP:		.RES	1		;VALUE OF E

612	00EE				NSIGN:	.RES	1		;SIGN OF #

613	00EF				ESIGN:	.RES	1		;SIGN OF EXPONENT

614	00F0				FCHRFLG:	.RES	1		;1ST CHAR FLAG

615	00F1				DIGRT:	.RES	1		;# OF DIGITS RIGHT OF	DECIMAL

616	00F2				CIX:		.RES	1		;CURRENT INPUT INDEX

617	00F3				INBUFF:	.RES	2		;POINTS TO USER'S LINE INPUT	BUFFER

618	00F5				ZTEMP1:	.RES	2

619	00F7				ZTEMP4:	.RES	2

620	00F9				ZTEMP3:	.RES	2

621	00FB				DEGFLG

622	00FB				RADFLG:	.RES	1		;0-RADIANS 6=DEGREES

623	0000				RADON		=	0		;INDICATES RADIANS

624	0006				DEGON		=	6		;INDICATES DEGREES

625	00FC				FLPTR:	.RES	2		;POINTS TO USER'S FLOATING PT	NUMBER

626	00FE				FPTR2:	.RES	2

627					; FLOATING PT ROUTINES' NON-ZERO PAGE RAM

62					; (NEEDED ONLY IF F. P. ROUTINES CALLED)

629								*=$57E

630	057E				LBPR1:	.RES	1		;LBUFF PREFIX 1

631	057E				LBPR2:	.RES	1		;LBUFF PREFIX 2

632	0580				LBUFF:	.RES	128		;LINE BUFFER

633	05EO				PLYARG	=	LBUFF+860	;POLYNOMIAL ARGUMENTS

634	05E6				FPSCR		=	PLYARG+FPREC

635	05EC				FPSCR1	=	FPSCR+FPREC

636	05E6				FSCR		=	FPSCR

637	05EC				FSCR1		=	FPSCRI

638	05FF				LBFEND	=	*-1	;END OF LBUFF

639					;

640					;

641					;

642					;

643					;

644					;

645					;

646					;

647					;

648					;	COLLEEN	MNEMONICS

649					;

650	D200				POKEY		=	$D200		;VBLANK ACTION:	DESCRIPTION:

651	D200				POT0		=	POKEY+0	;POT0-->PADDL0	0-227	IN RAM CELL

652	D201				POT1		=	POKEY+1	;POT1-->PADDL1	0-227	IN RAM CELL

653	D202				POT2		=	POKEY+2	;POT2-->PADDL2	0-227	IN RAM CELL

654	D203				POTS		=	POKEY+3	;POT3-->PADDL3	0-227	IN RAM CELL

655	D204				POT4		=	POKEY+4	;POT4-->PADDL4	0-227	IN RAM CELL

656	D205				POT5		=	POKEY+5	;POT5-->PADDL5	0-227	IN RAM CELL

657	D206				POT6		=	POKEY+6	;POT6-->PADDL6	0-227	IN RAM CELL

658	D207				POT7		=	POKEY+7	;POT7-->PADDL7	0-227 IN RAM CELL

659	D208				ALLPOT	=	POKEY+8	:???

660	D209				KBCODE	=	POKEY+9

661	D20A				RANDOM	=	POKEY+10

662	D20B				POTGO		=	POKEY+11	;STROBED

663	D20D				SERIN		=	POKEY+13

664	D20E				IRQST		=	POKEY+14

665	D20F				SKSTAT	=	POKEY+15

666	D200				AUDF1		=	POKEY+0

667	D201				AUDC1		=	POKEY+1

668	D202				AUDF2		=	POKEY+2

669	D203				AUDC2		=	POKEY+3

670	D204				AUDF3		=	POKEY+4

671	D205				AUDC3		=	POKEY+5

672	D206				AUDF4		=	POKEY+6

673	D207				AUDC4		=	POKEY+7

674	D208				AUDCTL	=	POKEY+B	;NONE	AUDCTL<--(SIO)

675	D209				STIMER	=	POKEY+9

676	D20A				SKRES		=	POKEY+10	;NONE	SKRES<--(SI0)

677	D20D				SEROUT	=	POKEY+13	;NONE	SEROUT<--CSI07

678	D20E				IRQEN		=	POKEY+14	;POKMSK-->IRQEN	(AFFECTED BY OPEN S: OR E:)

679	D20F				SKCTL		=	POKEY+15	;SSKCTL-->SKCTL	SSKCTL<--CSI07

680					;

681	D000				CTIA		=	$D000		;VBLANK ACTION:	DESCRIPTION:

682	D000				HPOSP0	=	CTIA+0

683	D001				HPOSP1	=	CTIA+1

684	D002				HPOSP2	=	CTIA+2

685	D003				HPOSP3	=	CTIA+3

686	D004				HPOSM0	=	CTIA+4

687	D005				HPOSM1	=	CTIA+5

688	D006				HPOSM2	=	CTIA+6

689	D007				HPOSM3	=	CTIA+7

690	D008				SIZEP0	=	CTIA+8

691	D009				SIZEP1	=	CTIA+9

692	D00A				SIZEP2	=	CTIA+10

693	D00B				SIZEP3	=	CTIA+11

694	D00C				SIZEM		=	CTIA+12

695	D00D				GRAFP0	=	CTIA+13

696	D00E				GRAFPI	=	CTIA+14

697	D00F				GRAFP2	=	CTIA+15

698	D010				GRAFP3	=	CTIA+16

699	D011				GRAFM		=	CTIA+17

700	D012				COLPMO	=	CTIA+18	;PCOLR0-->COLPM0	WITH ATTRACT MODE

701	D013				COLPMI	=	CTIA+19	;PCOLR1-->COLPMI	WITH ATTRACT MODE

702	D014				COLPM2	=	CTIA+20	:PCOLR2-->COLPM2	WITH ATTRACT MODE

703	D015				COLPM3	=	CTIA+21	;PCOLR3-->COLPM3	WITH ATTRACT MODE

704	D016				COLPF0	=	CTIA+22	;COLOR0-->COLPF0	WITH ATTRACT MODE

705	D017				COLPF1	=	CTIA+23	;COLOR1-->COLPF1	WITH ATTRACT MODE

706	D018				COLPF2	=	CTIA+24	;COLOR2-->COLPF2	WITH ATTRACT MODE

707	D019				COLPF3	=	CTIA+25	;COLOR3-->COLPF3	WITH ATTRACT MODE

708	D01A				COLBK		=	CTIA+26	;COLOR4-->COLBK	WITH ATTRACT MODE

709	D01B				PRIOR		=	CTIA+27	;(ON OPEN S: OR E	:) GPRIOR-->PRIOR

710	D01C				VDELAY	=	CTIA+28

711	D01D				GRACTL	=	CTIA+29

712	D01E				HITCLR	=	CTIA+30

713	D01F				CONSOL	=	CTIA+31	;*08-->CONSOL	TURN OFF SPEAKER

714	D000				M0PF		=	CTIA+0

715	D001				M1PF		=	CTIA+1

716	D002				M2PF		=	CTIA+2

717	D003				M3PF		=	CTIA+3

718	D004				P0PF		=	CTIA+4

719	D005				PlPF		=	CTIA+5

720	D006				P2PF		=	CTIA+6

721	D007				P3PF		=	CTIA+7

722	D008				M0PL		=	CTIA+8

723	D009				M1PL		=	CTIA+9

724	D00A				M2PL		=	CTIA+10

725	D00B				M3PL		=	CTIA+11

726	D00C				P0PL		=	CTIA+12

727	R00D				P1PL		=	CTIA+13

728	D00E				P2PL		=	CTIA+14

729	D00F				P3PL		=	CTIA+15

730	D010				TRIG0		=	CTIA+16	;TRIGO-->STRIGO

731	D011				TRIG1		=	CTIA+17	;TRIG1-->STRIGI

732	D012				TRIG2		=	CTIA+18	;TRIG2-->STRIG2

733	D013				TRIG3		=	CTIA+19	;TRIG3-->STRIG3

734					;

735	D400				ANTIC		=	$D400		;VBLANK ACTION	DESCRIPTION

736	D400				DMACTL	=	ANTIC+0	;DMACTL<--SDMCTL	ON OPEN S: OR E:

737	D401				CHACTL	=	ANTIC+1	;CHACTL<--CHACT	ON OPEN S: OR E:

738	D402				DLISTL	=	ANTIC+2	;DLISTL<--SDLSTL	ON OPEN S: OR E:

739	D403				DLISTH	=	ANTIC+3	;DLISTH<--SDLSTH	ON OPEN S: OR E:

740	D404				HSCROL		ANTIC+4

741	D405				VSCROL	=	ANTIC+5

742	D407				PMBASE	=	ANTIC+7

743	D409				CHBASE	=	ANTIC+9	;CHBASE<--CHBAS	ON OPEN S: OR E:

744	D40A				WSYNC		=	ANTIC+10

745	D40B				VCOUNT	=	ANTIC+11

746	D40C				PENH		=	ANTIC+12

747	D40D				PENV		=	ANTIC+13

748	D40E				NMIEN		=	ANTIC+14	;NMIEN<--40 POWER	ON AND CSETVBV7

749	D40F				NMIRES	=	ANTIC+15	;STROBED

750	D40F				NMIST		=	ANTIC+15

751	D300				PIA		=	$D300		;VBLANK ACTION	DESCRIPTION

752	D300				PORTA		=	PIA+0		;PORTA-->STICK0,1	X-Y CONTROLLERS

753	D301				PORTB		=	PIA+1		;PORTB-->STICK2.3	X-Y CONTROLLERS

754	D302				PACTL		=	PIA+2		;NONE	PACTL<--3C CINIT7

755	D303				PBCTL		=	PIA+3		;NONE	PBCTL<--3C CINIT7

756					;

757					;

758					;

759					;.PAGE

760 					;.PAGE

761 							LIST 	S