1586					; TITLE 'SIO ( SERIAL BUS INPUT/OUTPUT CONTROLLER )'

1587					; COLLEEN OPERATING SYSTEM

1588					;

1589					; SID ( SERIAL BUS INPUT/OUTPUT CONTROLLER )

1590					; WITH SOFTWARE BAUD RATE CORRECTION ON CASSETTE

1591					;

1592					;

1593					; AL MILLER 3-APR-79

1594					;

1595					;

1596					; THIS MODULE HAS ONE ENTRY POINT. IT IS CALLED BY THE DEVICE

1597					; HANDLERS. IT INTERPRETS A PREVIOUSLY ESTABLISHED DEVICE CONTROL

1598					; BLOCK (STORED IN GLOBAL RAM) TO ISSUE COMMANDS

1599					; TO THE SERIAL BUS TO CONTROL TRANSMITTING AND RECEIVING DATA.

1600					;

1601					;

1602					;

1603					;

1604					; 	.PAGE

1605					; 	 EQUATES

1606					;

1607					; DCD DEVICE BUS ID NUMBERS

1608	0030				FLOPPY 	=	$30

1609					; PRINTR 	=	$40

1610					; CASSET 	=	$60	; !!!!! *****

1611	0060				CASET		=	$60	; !!!!! *****

1612					;

1613					;

1614					; BUS COMMANDS

1615					;

1616	0052				READ 		=	'R

1617	0057				WRITE 	=	'W

1618					; STATIS	= 	'S

1619					; FORMAT 	= 	'!

1620					;

1621					;

1622					; COMMAND AUX BYTES

1623					;

1624	0053				SIDWAY	=	'S	; PRINT 16 CHARACTERS SIDEWAYS

1625	004E				NORMAL	=	'N	; PRINT 40 CHARACTERS NORMALLY

1626	0044				DOUBLE 	=	'D	; PRINT 20 CHARACTERS DOUBLE WIDE

1627	0050				PLOT 		=	'P	; PLOT MODE

1628					;

1629					;

1630					; BUS RESPONSES

1631					;

1632	0041				ACK 		=	'A	; DEVICE ACKNOWLEDGES INFORMATION

1633	004E				NACK 		=	'N	; DEVICE DID NOT UNDERSTAND

1634	0043				COMPLT 	=	'C	; DEVICE SUCCESSFULLY COMPLETED OPERATION

1635	0045				ERROR 	=	'E	; DEVICE INCURRED AN ERROR IN AN ATTEMPTED OP

1636					;

1637					;

1638					; MISCELLANEOUS	EQUATES

1639					;

1640	0028				B192LO 	=	$28	; 19200 BAUD RATE POKEY COUNTER VALUES (LO BYTE)

1641	0000				B192HI 	=	$00	; 19200 BAUD (HI BYTE)

1642	00CC				B600LO 	=	$CC	; 600 BAUD (LO BYTE)

1643	0005				B600HI 	=	$05	; 600 BAUD (HI BYTE)

1644	0005				HITONE 	=	$05	; FSK HI FREQ POKEY COUNTE VALUE (5326 HZ)

1645	0007				LOTONE 	=	$07	; FSK LO FREQ POKEY COUNTER VALUE (3995 HZ)

1646					;

1647					.IF	PALFLG

1648					WIRGLO 	=	150	; WRITE INTER RECORD GAP (IN 1/60 SEC)

1649					RIRGLO 	=	100	; READ INTER RECORD GAP (IN 1/60 SEC)

1650					WSIRG 	=	 13	; SHORT WRITE INTER RECORD GAP

1651					RSIRG 	=	  8	; SHORT READ INTER RECORD GAP

1652					.ENDIF

1653					.IF	PALFLG-1

1654	00B4				WIRGLO 	=	180	; WRITE INTER RECORD GAP (IN 1/60 SEC)

1655	0078				RIRGLO 	=	120	; READ INTER RECORD GAP (IN 1/60 SEC)

1656	000F				WSIRG 	=	15	; SHORT WRITE INTER RECORD GAP

1657	000A				RSIRG 	=	10	; SHORT READ INTER RECORD GAP

1658					.ENDIF

1659	0000				WIRGHI 	=	0

1660	0000				RIRGHI 	=	0

1661

1662	0034				NCOMLO 	=	$34 		; PIA COMMAND TO LOWER NOT COMMAND LINE

1663	003C				NCOMHI 	=	$3C 		; PIA COMMAND TO RAISE NOT COMMAND LINE

1664	0034				MOTRGO 	=	$34 		; PIA COMMAND TO TURN ON CASSETTE MOTOR

1665	003C				MOTRST 	=	$3C 		; PIA COMMAND TO TURN OFF MOTOR

1666					;

1667	0002				TEMPHI	=	TEMP/256 	; ADDRESS OF TEMP CELL (HI BYTE)

1668	003E				TEMPLO 	=	(-256)*TEMPHI+TEMP 	; (LO BYTE)

1669	0002				CBUFHI 	=	CDEVIC/256 	; ADDRESS OF COMMAND BUFFER (HI BYTE)

1670	003A				CBUFLO 	=	(-256)*CBUFHI+CDEVIC 	; (LO BYTE)

1671					;

1672	000D				CRETRI 	=	13  		; NUMBER OF COMMAND FRAME RETRIES

1673	0001				DRETRI 	=	1 		; NUMBER OF DEVICE RETRIES

1674	0002				CTIMLO 	=	2 		; COMMAND FRAME ACK TIME OUT (LO BYTE)

1675	0000				CTIMHI 	=	0 		; COMMAND FRAME ACK TIME OUT (HI BYTE)

1676					;

1677					;

1678					; JTADRH =	JTIMER/256 ; HI BYTE OF JUMP TIMER ROUTINE ADDR "M

1679					; JTADRL =	(-256)*JTADRH+JTIMER ; "MOVED TO LINE 1428"

1680					;

1681					.PAGE

1682					; SIO

1683					;

1684					;

1685							*=SIOV

1686	E459	4C 59	E9				JMP 	SIO		; SIO ENTRY POINT

1687					;

1688							*=SIOINV

1689	E465	4C 44	E9				JMP 	SIOINT	; SIO INITIALIZATION ENTRY POINT

1690					;

1691							*=SENDEV

1692	E468	4C F2	EB				JMP	 SENDEN	; SEND ENABLE ENTRY POINT

1693

1694							*=VCTABL-INTABS+VSERIN

1695

1696	E48A	0F EB					.WORD ISRSIR	; VSERIN

1697	E48C	90 EA					.WORD ISRODN	; VSEROR

1698	E48E	CF EA					.WORD ISRTD		; V SEROC

1699					;

1700					;

1701					;

1702							*=SIOORG

1703					;

1704					; SIO INITIALIZATION SUBROUTINE

1705					;

1706	E944	A9 3C			SIOINT: 	LDA 	#MOTRST

1707	E946	8D 02	D3				STA 	PACTL		; TURN OFF MOTOR

1708

1709	E949	A9 3C					LDA 	#NCOMHI

1710	E94B	BD 03	D3				STA 	PBCTL	:RAISE NOT COMMAND LINE

1711

1712

1713	E94E	A9 03					LDA 	#3

1714	E950	8D 32	02				STA 	SSKCTL	; GET POKEY OUT OF INITIALIZE MODE

1715	E953	85 41					STA 	SOUNDR	; INIT POKE ADDRESS FOR QUIET I/0

1716	E955	8D 0F	D2				STA 	SKCTL

1717					;

1718					;

1719	E958	60					RTS	:RETURN

1720					;

1721					;

1722					;

1723					;

1724					;

1725					;

1726	E959	BA			SIO: 		TSX

1727	E95A	BE 18	03				STX 	STACKP	; SAVE STACK POINTER

1728	E95D	A9 01					LDA 	#1

1729	E95F	85 42					STA 	CRITIC

1730					;

1731	E961	AD 00	03				LDA 	DDEVIC

1732	E964	C9 60					CMP 	#CASET

1733	E966	D0 03					BNE 	NOTCST	; BRANCH IF NOT CASSETTE

1734	E968	4C 80	EB				JMP 	CASENT	; OTHERWISE JUMP TO CASSETTE ENTER

1735					;

1736					; ALL DEVICES	EXCEPT CASSETTE ARE INTELLIGENT

1737					;

1738	E96B	A9 00			NOTCST:	LDA	40

1739	E96D	8D 0F	03				STA	CASFLG	; INIT CASSETTE FLAG TO NO CASSETTE

1740					;

1741	E970	A9 01					LDA	BDRETRI	; SET NUMBER OF DEVICE RETRIES

1742	E972	85 37					STA	DRETRY

1743	E974	A9 0D			COMMND:	LDA	#CRETRI	; SET NUMBER OF COMMAND FRAME RETRIES

1744	E976	85 36					STA	CRETRY

1745					;

1746					; SEND	A COMMAND FRAME

1747					;

1748	E978	A9 28			COMFRM:	LDA	#B192LO	; SET BAUD RATE TO 19200

1749	E97A	8D 04	D2				STA	AUDF3

1750	E97D	A9 00					LDA	#B192HI

1751	E97F	8D 06	D2				STA	AUDF4

1752					;

1753	E982	18					CLC			; SET UP COMMAND BUFFER

1754	E983	AD 00	03				LDA	DDEVIC

1755	E986	6D 01	03				ADC	DUNIT

1756	E989	69 FF					ADC	4fFF	; SUBTRACT 1

1757	E98B	8D 3A	02				STA	CDEVIC	; SET BUS ID NUMBER

1758					;

1759	E98E	AD 02	03				LDA	DCOMND

1760	E991	8D 3B	02				STA	CCOMND	; SET BUS COMMAND

1761					;

1762	E994	AD 0A	03				LDA	DAUX1		; STORE COMMAND FRAME AUX BYTES 1 AND 2

1763	E997	8D 3C	02				STA	CAUX1

1764	E99A	AD 0B	03				LDA	DAUX2

1765	E99D	8D 3D	02				STA	CAUX2		; DONE SETTING UP COMMAND BUFFER

1766					;

1767	E9A0	18					CLC	 		; SET BUFFER POINTER TO COMMAND FRAME BUFFER

1768	E9A1	A9 3A					LDA	#CBUFLO

1769	E9A3	85 32					STA	BUFRLO		; AND BUFFER END ADDRESS

1770	E9A5	69 04					ADC	44

1771	E9A7	85 34					STA	BFENLO

1772	E9A9	A9 02					LDA	#CBUFHI

1773	E9AB	85 33					STA	BUFRHI

1774	E9AD	85 35					STA	BFENHI	; DONE SETTING UP BUFFER POINTER

1775					;

1776	E9AF	A9 34					LDA	#NCOMLO	; LOWER NOT COMMAND LINE

1777	E9B1	8D 03	D3				STA	PBCTL

1778					;

1779	EqB4	20 BA	EC				JSR	SENDIN	; SEND THE COMMAND FRAME TO A SMART DEVICE

1780					;

1781	E9B7	AD 3F	02				LDA	ERRFLG

1782	E9BA	D0 03					BNE	BADCOM	; BRANCH IF AN ERROR RECEIVED

1783					;

1784	E9BC	98					TYA

1785	E9BD	D0 07					BNE	ACKREC	; BRANCH IF ACK RECEIVED

1786					;

1787					;

1788	E9BF	C6 36			BADCOM:	DEC	CRETRY	; A NACK OR TIME OUT OCCURED

1789	E9C1	10 85					BPL	COMFRM	; SO BRANCH IF ANY RETRIES LEFT

1790					;

1791	E9C3	4C 06	EA				JMP	DERR1		; OTHERWISE. JUMP TO RETURN SECTION

1792					;

1793					;

1794	E9C6	AD 03	03		ACKREC:	LDA	DSTATS	; ACK WAS RECEIVED

1795	E9C9	10 0C					BPL	WATCOM	; BRANCH TO WAIT FOR COMPLETE .

1796					; IF THERE IS	NO DATA TO BE	SENT

1797					;

1798					;

1799					;

1800					; SEND A DATA FRAME TO PERIPHERAL

1801					;

1802	E9CB	A9 0D					LDA	#CRETRI	; SET NUMBER OF RETRIES

1803	E9CD	85 36					STA	CRETRY

1804					;

1805	E9CF	20 6A	EB				JSR	LDPNTR	; LOAD BUFFER POINTER WITH DCB INFORMATION

1806					;

1807	E9D2	20 BA	EC				JSR	SENDIN	; GO SEND THE DATA FRAME TO A SMART DEVICE

1808					;

1809	E9D5	F0 E8					BEQ	BADCOM	; BRANCH IF BAD

1810					;

1811					;

1812					;

1813					; WAIT FOR COMPLETE SIGNAL FROM PERIPHERAL

1814					;

1815	E9D7	20 75	EC		WATCOM:	JSR	STTMOT	; SET DDEVICE TIME OUT VALUES IN Y~X

1816					;

1817	E9DA	A9 00					LDA	#800

1818	E9DC	BD 3F	02				STA	ERRFLG	; CLEAR ERROR FLAG

1819					;

1820	E9DF	20 9B	EC				JSR	WAITER	; SET UP TIMER AND WAIT

1821	E9E2	F0 12					BEQ	DERR		; BRANCH IF TIME OUT

1822					;

1823					;

1824					; DEVICE DID NOT TIME OUT

1825					;

1826	E9E4	2C 03	03				BIT	OSTATS

1827	E9E7	70 07					BVS	MODATA	; BRANCH IF MORE DATA FOLLOWS

1828					;

1829	E9E9	AD 3F	02				LDA	ERRFLG

1830	E9EC	D0 18					BNE	DERR1		; BRANCH IF AN ERROR OCCURRED

1831	E9EE	F0 1D					BEQ	RETURN	; OTHERWISE RETURN

1832					;

1833					;

1834					;

1835					;

1836  				; RECEIVE A DATA FRAME FROM PERIPHERAL                                                                                                                                                                                                                                                                                                	1837					;

1838	E9F0	20 6A	EB		MODATA:	JSR	LDPNTR	; LOAD BUFFER POINTER WITH DCB INFORMATION

1839					;

1840	E9F3	20 E0	EA				JSR	RECEIV	; GO RECEIVE A DATA FRAME

1841					;

1842	E9F6	AD 3F	02		DERR:		LDA	ERRFLG

1843	E9F9	F0 05					BEQ	NOTERR	; BRANCH IF NO ERROR PRECEEDED DATA

1844					;

1845	E9FB	AD 19	03				LDA	TSTAT		; GET TEMP STATUS

1846	E9FE	85 30					STA	STATUS	; STORE IN REAL STATUS

1847					;

1848					;

1849	EA00	A5 30			NOTERR:	LDA	STATUS

1850	EA02	C9 01					CMP	#SUCCES

1851	EA04	F0 07					BEQ	RETURN	; BRANCH IF COMPLETELY SUCCESSFUL

1852					;

1853	EA06	C6 37			DERR1:	DEC	DRETRY

1854	EA08	30 03					BMI	RETURN	; BRANCH If OUT OF DEVICE RETRIES

1855					;

1856	EA0A	4C 74	E9				JMP	COMMND	; OTHERWISE. ONE MORE TIME

1857					;

1858					;

1859					;

1860					;

1861	EA0D	20 5F	EC		RETURN:	JSR	SENDDS	; DISABLE POKEY INTERRUPTS

1862	EA10	A9 00					LDA	*0

1863	EA12	85 42					STA	CRITIC

1864	EA14	A4 30					LDY	STATUS	; RETURN STATUS IN Y

1865	EA16	8C 03	03	 			STY	DSTATS	; AND THE DCB STATUS WORD

1866	EA19	60					RTS	RETURN

1867					;

1868					;

1869					;

1870					;

1871					; WAIT SUBROUTINE

1872					;

1873					; WAITS FOR COMPLETE OR ACK

1874					; RETURNS Y=$FF	IF SUCCESSFUL Y=$00 IF NOT

1875					;

1876	EA1A	A9 00			WAIT:		LDA	#$00

1877	EAlC	8D 3F	02				STA	ERRFLG	; CLEAR ERROR FLAG

1878					;

1879	EAlF	18					CLC			; LOAD BUFFER POINTER WITH ADDRESS

1880	EA20	A9 3E					LDA	#TEMPLO	; OF TEMPORARY RAM CELL

1881	EA22	85 32					STA	BUFRLO

1882	EA24	69 01					ADC	#1

1883	EA26	85 34					STA	BFENLO	; ALSO SET BUFFER END +1 ADDRESS

1884	EA28	A9 02					LDA	#TEMPHI

1885	EA2A	85 33					STA	BUFRHI

1886	EA2C	85 35					STA	BFENHI	; DONE LOADING POINTER

1887					;

1888	EA2E	A9 FF					LDA	#$FF

1889	EA30	85 3C					STA	NOCKSM	; SET NO CHECKSUM FOLLOWS DATA FLAG

1890					;

1891	EA32	20 E0	EA				JSR	RECEIV	; GO RECEIVE A BYTE

1892					;

1893	EA35	A0 FF					LDY	#$FF		; ASSUME SUCCESS

1894	EA37	A5 30					LDA	STATUS

1895	EA39	C9 01					CMP	#SUCCES

1896	EA3B	D0 19					BNE	NWOK		; BRANCH IF IT DID NOT WORK OK

1897					;

1898					;

1899					;

1900					;

1901	EA3D	AD 3E	02		WOK:		LDA	TEMP		; MAKE SURE THE BYTE SUCCESSFULLY RECEIVED

1902	EA40	C9 41					CMP	#ACK		; WAS ACTUALLY AN ACK OR COMPLETE

1903	EA42	F0 21					BEQ	GOOD

1904	EA44	C9 43					CMP	#COMPLT

1905	EA46	F0 1D					BEQ	GOOD

1906					;

1907	EA48	C9 45					CMP	#ERROR

1908	EA4A	D0 06					BNE	NOTDER	; BRANCH IF DEVICE DID NOT SEND BACK

1909					; A DEVICE ERROR CODE

1910	EA4C	A9 90					LDA	#DERROR

1911	EA4E	85 30					STA	STATUS	; SET DEVICE ERROR STATUS

1912	EA50	D0 04					BNE	NWOK

1913

1914	EA52	A9 8B			NOTDER:	LDA	#DNACK	; OTHERWISE SET NACK STATUS

1915	EA54	85 30					STA	STATUS

1916					;

1917	EA56	A5 30			NWOK:		LDA	STATUS

1918	EA58	C9 8A					CMP	#TIMOUT

1919	EA5A	F0 07					BEQ	BAD		; BRANCH IF TIME OUT

1920					;

1921	EA5C	A9 FF					LDA	#$FF

1922	EA5E	8D 3F	02				STA	ERRFLG	; SET SOME ERROR FLAG

1923	EA61	D0 02					BNE	GOOD		; RETURN WITH OUT SETTING Y = 0

1924					;

1925	EA63	A0 00			BAD:		LDY	#0

1926					;

1927	EA65	A5 30			GOOD:		LDA	STATUS

1928	EA67	8D 19	03				STA	TSTAT

1929	EA6A	60					RTS			; RETURN

1930					;

1931					;

1932					;

1933					;

1934					;

1935					; SEND	SUBROUT	INE

1936					;

1937					; SENDS	A BUFFER OF BYTES	OUT OVER THE SERIAL BUS

1938					;

1939					;

1940	EA6B	A9 01			SEND:		LDA	#SUCCES	; ASSUME SUCCESS

1941	EA61)	85 30					STA	STATUS

1942					;

1943	EA6F	20 F2	EB				JSR	SENDEN	; ENABLE SENDING

1944					;

1945	EA72	A0 00					LDY	#0

1946	EA74	84 31					STY	CHKSUM	; CLEAR CHECK SUM

1947	EA76	84 3B					STY	CHKSNT	; CHECKSUM SENT FLAG

1948	EA78	84 3A					STY	XMTDON	;TRANSMISSION DONE FLAG

1949					;

1950					;

1951	EA7A	B1 32					LDA	(BUFRLO),Y	; PUT FIRST BYTE FROM BUFFER

1952	EA7C	SD 0D	D2				STA	SEROUT	; INTO THE SERIAL OUTPUT REGISTER

1953					;

1954					;

1955	EA7F	85 31					STA	CHKSUM	;PUT IT IN CHECKSUM

1956					;

1957	EA81	A5 11			NOTDON:	LDA	BRKKEY

j1958	EA83	D0 03					BNE	NTBRKO

1959	EA85	4C A0	ED				JMP	BROKE		; JUMP IF BREAK KEY PRESSED

1960					;

1961	EA88	A5 3A			NTBRKO:	LDA	XMTDON	; LOOP UNTIL TRANSMISSION IS DONE

1962	EASA	F0 F5					BEQ	NOTDON

1963					;

1964	EASC	20 5F	EC				JSR	SENDDS	; DISABLE SENDING

1965					;

1966	EASF	60					RTS			; RETURN

1967					;

1968					;

1969					;

1970					;

1971					;

1972					;

1973					; OUTPUT DATA NEEDED INTERRUPT SERVICE ROUTINE

1974					;

1975	EA90	98			ISRODN:	TYA

1976	EA91	48					PHA			; SAVE Y REG ON STACK

1977					;

1978	EA92	E6 32					INC	BUFRLO	; INCREMENT BUFFER POINTER

1979	EA94	D0 02					BNE	NOWRPO

1980	EA96	E6 33					INC	BUFRHI

1981					;

1982	EA98	A5 32			NOWRPO:	LDA	BUFRLO	; CHECK IF PAST END OF BUFFER

1983	EA9A	C5 34					CMP	BFENLO

1984	EA9C	A5 33					LDA	BUFRHI	; HIGH PART

1985	EA9E	E5 35					SBC	BFENHI

1986	EAA0	90 1C					BCC	NOTEND	; BRANCH IF NOT PAST END OF BUFFER

1987					;

1988	EAA2	A5 3B					LDA	CHKSNT

1989	EAA4	D0 0B					BNE	RELONE	; BRANCH IF CHECKSUM ALREADY SENT

1990					;

1991	EAA6	A5 31					LDA	CHKSUM

1992	EAAS	SD 0D	D2				STA	SEROUT	; SEND CHECK SUM

1993	EAAB	A9 FF					LDA	#SFF

1994	EAAD	85 3B					STA	CHKSNT	; SET CHECKSUM SENT FLAG

1995	EAAF	D0 09					BNE	CHKDON

1996					;

1997	EAB1	A5 10			RELONE:	LDA	POKMSK	; ENABLE TRANSMIT DONE INTERRUPT

1998	EAB3	09 08					ORA	#$08

1999	EAB5	85 10					STA	POKMSK

2000	EAB7	SD 0E	D2				STA	IRQEN

2001		 			;

2002	EABA	68			CHKDON:	PLA

2003	EABB	AS					TAY			; RESTORE Y REG

2004	EABC	68					PLA			; RETURN FROM INTERRUPT

2005	EABD	40					RTI

2006					;

2007					;

2008	EABE	A0 00			NOTEND:	LDY	#0

2009	EAC0	B1 32					LDA	(BUFRLO),Y	; PUT NEXT BYTE FROM BUFFER

2010	EAC2	8D 0D	D2				STA	SEROUT	; INTO THE SERIAL OUTPUT REGISTER

2011					;

2012	EAC5	18					CLC			; ADD IT TO CHECKSUM

2013	EAC6	65 31					ADC	CHKSUM

2014	EAC8	69 00					ADC	#0

2015	EACA	85 31					STA	CHKSUM

2016					;

2017	EACC	4C BA	EA				JMP	CHKDON	; GO RETURN

2018					;

2019					;

2020					;

2021					;

2022					;

2023					;

2024					; TRANSMIT DONE INTERRUPT SERVICE ROUTINE

2025					;

2026	EACF	A5 3B			ISRTD:	LDA	CHKSNT

2027	EAD1	F0 0B					BEQ	FOOEY		; BRANCH IF CHECKSUM NOT YET SENT

2028					;

2029	EAD3	85 3A					STA	XMTDON	; OTHERWISE SET TRANSMISSION DONE FLAG

2030					;

2031	EAD5	A5 10					LDA	POKMSK	; DISABLE TRANSMIT DONE INTERRUPT

2032	EAD7	29 F7					AND	#$F7

2033	EAD9	85 10					STA	POKMSK

2034	EADB	8D 0E	D2				STA	IRQEN

2035

2036	EADE	68			FOGEY:	PLA			; RETURN FROM INTERRUPT

2037	EADF	40					RTI

2038					;

2039					;

2040					;

2041					;

2042					;

2043					;

2044					;

2045					;

2046					; RECEIVE SUBROUTINE

2047					;

2048	EAE0	A9 00			RECEIV:	LDA	#0

2049					;

2050	EAE2	AC 0F	03				LDY	CASFLG

2051	EAE5	D0 02					BNE	NOCLR		; BRANCH IF CASSETTE

2052					;

2053	EAE7	85 31					STA	CHKSUM	; CLEAR CHKSUM

2054	EAE9	85 38			NOCLR:	STA	BUFRFL	; BUFFER FULL FLAG

2055	EAEB	85 39					STA	RECVDN	; RECEIVE DONE FLAG

2056					;

2057					;

2058					;

2059	EAED	A9 01					LDA	#SUCCES

2060	EAEF	85 30					STA	STATUS	; SET GOOD STATUS FOR DEFAULT CASE.

2061	EAF1	20 1B	EC				JSR	RECVEN	; DO RECEIVE ENABLE

2062	EAF4	A9 3C					LDA	ANCOMHI	; COMMAND FRAME HI COMMAND

2063	EAF6	8D 03	D3				STA	PBCTL		; STORE IN PIA

2064	EAF9	A5 11			CHKTIM:	LDA	BRKKEY

2065	EAFB	D0 03					BNE	NTBRK1

2066	EAFD	4C A0	ED				JMP	BROKE		; JUMP IF BREAK KEY PRESSED

2067					;

2068	EB00	AD 17	03		NTBRKI:	LDA	TIMFLG	; NO.

2069	EB03	F0 05					BEQ	TOUT		; IF TIMEOUT~ GO SET ERROR STATUS

2070	EB05	A5 39					LDA	RECVDN

2071	EB07	F0 F0					BEQ	CHKTIM	; DONE ?

2072	EB09	60 			GOBACK:	RTS

2073	EB0A	A9 8A			TOUT:		LDA	#TIMOUT	; YES.

2074	EB0C	85 30					STA	STATUS	; SET TIMEOUT STATUS

2075					;

2076					;

2077					;

2078					;

2079					;

2080					;

2081	EB0E	60			RRETRN:	RTS			; RETURN

2082					;

2083					;

2084					;

2085					;

2086					;

2087					;

2088					;

2089					; SERIAL INP	UT READY INTERRUPT SERVICE ROUTINE

2090					;

2091	EB0F	98			ISRSIR:	TYA

2092	EB10	48					PHA			; SAVE Y REG ON STACK

2093					;

2094					;

2095					;

2096	EB11	AD 0F	D2				LDA	SKSTAT

2097	EB14	8D 0A	D2				STA	SKRES		; RESET STATUS REGISTER

2098					; ******** THIS MAY NOT	BE THE PLACE TO DO IT ********

2099					;

2100	EB17	30 04					BMI	NTFRAM	; BRANCH IF NO FRAMING ERROR

2101					;

2102	EB19	A0 8C					LDY	#FRMERR

2103	EB1B	84 30					STY	STATUS	; SET FRAME ERRORR STATUS

2104					;

2105	EB1D	29 20			NTFRAM:	AND	#x20

2106	EB1F	D0 04					BNE	NTOVRN	; BRANCH IF NO OVERRUN ERROR

2107					;

2108	EB21	A0 8E					LDY	#OVRRUN

2109	EB23	84 30					STY	STATUS	; SET OVERRUN ERROR STATUS

2110					;

2111	EB25	A5 38			NTOVRN:	LDA	BUFRFL

2112	EB27	F0 13					BEQ	NOTYET	; BRANCH IF BUFFER WAS NOT YET FILLED

2113					;

2114	EB29	AD 0D D2				LDA	SERIN		; THIS INPUT BYTE IS THE CHECKSUM

2115	EB2C	C5 31					CMP	CHKSUM

2116	EB2E	F0 04					BEQ	SRETRN	; BRANCH IF CHECKSUMS MATCH

2117					;

2118	EB30	A0 8F					LDY	*CHKERR

2119	EB32	84 30					STY	STATUS	; SET CHECKSUM ERROR STATUS

2120					;

2121	EB34	A9 FF			SRETRN:	LDA	#$FF		; SET RECEIVE DONE FLAG

2122	EB36	85 39					STA	RECVDN

2123					;

2124	EB38	68 			SUSUAL:	PLA

2125	EB39	A8 					TAY			; RESTORE Y REG

2126	EB3A	68					PLA			; RETURN FROM INTERRUPT

2127	E83B	40					RTI

2128					;

2129					;

2130					;

2131	EB3C	AD 0D D2		NOTYET:	LDA	SERIN

2132	EB3F	A0 00					LDY	#0

2133	E841	91 32					STA	(BUFRLO),Y	; STORE INPUT REGISTER INTO BUFFER

2134					;

2135	EB43	18					CLC			; ADD IT TO CHECKSUM

2136	EB44	65 31					ADC	CHKSUM

2137	EB46	69 00					ADC	#0

2138	EB48	85 31					STA	CHKSUM

2139					;

2140	EB4A	E6 32					INC	BUFRLO	; INCREMENT BUFFER POINTER

2141	E84C	D0 02					BNE	NTWRP1

2142	EB4E	E6 33					INC	BUFRHI

2143					;

2144	E850	A5 32			NTWRPI:	LDA	BUFRLO

2145	EB52	C5 34					CMP	BFENLO

2146	E854	A5 33					LDA	BUFRHI

2147	EB56	E5 35					SBC	BFENHI

2148	EB58	90 DE					BCC	SUSUAL	; BRANCH IF NEW BUFFER ADDRESS IS IN BUFFER L

2149					;

2150	EB5A	A5 3C					LDA	NOCKSM

2151	EB5C	F0 06					BEQ	GOON		; BRANCH IF A CHECKSUM WILL FOLLOW DATA

2152					;

2153	EB5E	A9 00					LDA	110

2154	EB60	85 3C					STA	NOCKSM	; CLEAR NO CHECKSUM FLAG

2155					;

2156	EB62	F0 D0					BEQ	SRETRN	; GO RETURN AND SET RECEIVE DONE FLAG

2157					;

2158					;

2159	EB64	A9 FF			GOON:		LDA	#$FF

2160	EB66	85 38					STA	BUFRFL	; SET BUFFER FULL FLAG

2161					;

2162	EB68	D0 CE					BNE	SUSUAL	; GO RETURN

2163					;

2164					;

2165					;

2166					;

2167					;

2168					;

2169					;

2170					;

2171					; LOAD BUFFER POINTER SUBROUTINE

2172					;

2173					; LOAD BUFFER POINTER WITH DCB BUFFER INFORMATION

2174					;

2175	EB6A	18			LDPNTR:	CLC

2176	EB6B	AD 04	03				LDA	DBUFLO

2177	EB6E	85 32					STA	BUFRLO

2178	EB70	6D 08	03				ADC	DBYTLO

2179	EB73	85 34					STA	BFENLO	; ALSO SET BUFFER END + 1 ADDRESS

2180					;

2181	EB75	AD 05	03				LDA	DBUFHI

2182	EB78	85 33					STA	BUFRHI

2183	EB7A	6D 09	03				ADC	DBYTHI

2184	EB7D	85 35					STA	BFENHI

2185					;

2186	EB7F	60					RTS			; RETURN

2187					;

2188					;

2189					;

2190					;

2191					;

2192					;

2193					;

2194					;

2195					; CASSETTE HANDLING CODE

2196					;

2197	EB80	AD 03	03		CASENT:	LDA	DSTATS

2198	EB83	10 2E					BPL	CASRED	; BRANCH IF INPUT FROM CASSETTE

2199					;

2200					; WRITE	A RECORD

2201					;

2202	EB85	A9 CC					LDA	#B600LO	; SET BAUD RATE TO 600

2203	EB87	8D 04	D2				STA	AUDF3

2204	EB8A	A9 05					LDA	#B600HI

2205	EBBC	8D 06	D2				STA	AUDF4

2206					;

2207	EBBF	20 F2	EB				JSR	SENDEN	; TURN ON POKEY MARK TONE

2208					;

2209	EB92	A0 0F					LDY	#WSIRG	; LOAD SHORT WRITE INTER RECORD GAP TIME

2210	EB94	AD 0B	03				LDA	DAUX2

2211	EB97	30 02					BMI	SRTIRO	; BRANCH IF SHORT GAP IS DESIRED

2212					;

2213	EB99	A0 B4					LDY	#WIRGLO	; SET WRITE IRG TIME

2214	EB9B	A2 00			SRTIRO:	LDX	#WIRGHI

2215	EB9D	20 B9	ED				JSR	SETVBX

2216					;

2217	EBA0	A9 34					LDA	#MOTRGO

2218	EBA2	8D 02	D3				STA	PACTL		; TURN ON MOTOR

2219					;

2220	EBA5	AD 17	03		TIMIT:	LDA	TIMFLG	; LOOP UNTIL DONE

2221	EBAB	D0 FB					BNE	TIMIT

2222					;

2223	EBAA	20 6A	EB				JSR	LDPNTR	; LOAD BUFFER POINTER WITH DCB INFORMATION

2224					;

2225	EBAD	20 6B	EA				JSR	SEND		; SEND A BUFFER

2226					;

2227	EBB0	4C DF	EB				JMP	CRETRN	; GO RETURN

2228					;

2229					;

2230					;

2231					; RECEIVE A	RECORD

2232					;

2233	EBB3	A9 FF			CASRED:	LDA	#$FF

2234	EBB5	8D 0F	03				STA	CASFLG	; SET SET CASSETTE FLAG

2235					;

2236	EBBS	A0 0A					LDY	#RSIRG	;LOAD SHORT READ INTER RECORD GAP TIME

2237	EBBA	AD 0B	03				LDA	DAUX2

2238	EBBD	30 02					BMI	SRTIRI	; BRANCH IF SHORT GAP IS DESIRED

2239

2240	EBBF	A0 78					LDY	#RIRGLO	; SET TIME OUT FOR READ IRG

2241	EBC1	A2 00			SRTIRI:	LDX	#RIRGHI

2242	EBC3	20 B9	ED				JSR	SETVBX

2243					;

2244	EBC6	A9 34					LDA	#MOTRGO

2245	EBCB	8D 02	D3				STA	PACTL		; TURN ON MOTOR

2246					;

2247	EBCB	AD 17	03		TIMIT1:	LDA	TIMFLG	; LOOP UNTIL DONE

2248	EBCE	D0 FB					BNE	TIMIT1

2249					;

2250	EBD0	20 6A	EB				JSR	LDPNTR	; LOAD BUFFER POINTER WITH DCB INFORMATION

2251					;

2252	EBD3	20 75	EC				JSR	STTMOT	; SET DEVICE TIME OUT IN Y,X

2253	EBD6	20 B9	ED				JSR	SETVBX

2254					;

2255	EBD9	20 10	ED				JSR	BEGIN		; SET INITIAL BAUD RATE

2256					;

;

2257	EBDC	20 E0	EA				JSR	RECEIV	; GO RECEIVE A BLOCK

2258

2259	EBDF	AD 0B	03		CRETRN:	LDA	DAUX2

2260	EBE2	30 05					BMI	SRTIR2	; BRANCH IF DOING SHORT INTER RECORD GAPS

2261					; DON'T TURN OFF CASSETTE MOTOR

2262	EBE4	A9 3C					LDA	#MOTRST

2263	EBE6	8D 02	D3				STA	PACTL		; TURN OFF MOTOR

2264					;

2265	EBE9	4C 0D	EA		SRTIR2:	JMP	RETURN	; GO RETURN

2266					;

2267					;

2268					;

2269					;

2270					;

2271	EBEC	A9 00			JTIMER:	LDA	#$00

2272	00EB				JTADRH	=	JTIMER/256	; HI BYTE OF JUMP TIMER ROUTINE ADDR

2273	00EC				JTADRL	=	(-256)*JTA	DRH+JTIMER

2274	EBEE	BD 17	03				STA	TIMFLG	; SET TIME OUT FLAG

2275	EBF1	60					RTS

2276					;

2277					;

2278					;

2279					;

2280					;

2281					;

2282					; SEND	ENABLE	SUBROUTINE

2283					;

2284	EBF2	A9 07			SENDEN:	LDA	#$07		; MASK OFF PREVIOUS SERIAL BUS CONTROL BITS

2285	EBF4	2D 32	02				AND	SSKCTL

2286	EBF7	09 20					ORA	#$20		; SET TRANSMIT MODE

2287					;

2288	EBF9	AC 00	03				LDY	DDEVIC

2289	EBFC	C0 60					CPY	#CASET

2290	EBFE	D0 0C					BNE	NOTCAS	; BRANCH IF NOT CASSETTE

2291					;

2292	EC00	09 08					ORA	#$08		; SET THE FSK OUTPUT BIT

2293					;

2294	EC02	A0 07					LDY	#LOTONE	; SET FSK TONE FREQUENCIES

2295	EC04	8C 02	D2				STY	AUDF2

2296	EC07	A0 05					LDY	#HITONE

2297	EC09	SC 00	D2				STY	AUDF1

2298					;

2299	EC0C	8D 32	02		NOTCAS:	STA	SSKCTL	; STORE NEW VALUE TO SYSTEM MASK

2300	EC0F	8D 0F	D2				STA	SKCTL		; STORE TO ACTUAL REGISTER

2301

2302	EC12	A9 C7					LDA	#$C7		; MASK OFF PREVIOUS SERIAL BUS INTERRUPT BITS

2303	EC14	25 10					AND	POKMSK

2304	EC16	09 10					ORA	#$10	; ENABLE OUTPUT DATA NEEDED INTERRUPT

2305					;

2306					;

2307	EC18	4C 31	EC				JMP	CONTIN	; GO CONTINUE IN RECEIVE ENABLE SUBROUTINE

2308					;

2309					;

2310					;

2311					;

2312					;

2313					;

2314					;

2315					;

2316					;

2317					;

2318					; RECEIVE ENABLE SUBROUTINE

2319					;

2320	EC1B	A9 07			RECVEN:	LDA	#$07		; MASK OFF PREVIOUS SERIAL BUS CONTROL BITS

2321	EC1D	2D 32	02				AND	SSKCTL

2322	EC20	09 10					ORA	#$10		; SET RECEIVE MODE ASYNCH.

2323	EC22	8D 32	02				STA	SSKCTL	; STORE NEW VALUE TO SYSTEM MASK

2324	EC25	8D 0F	D2				STA	SKCTL		; STORE TO ACTUAL REGISTER

2325					;

2326	EC28	8D 0A	D2				STA	SKRES		; RESET SERIAL PORT/KEYBOARD STATUS REGISTER

2327					;

2328	EC2B	A9 C7					LDA	#$C7		; MASK OFF PREVIOUS SERIAL BUS INTERRUPT BITS

2329	EC2D	25 10					AND	POKMSK

2330	EC2F	09 20					ORA	#$20		; ENABLE RECEIVE INTERRUPT

2331	EC31	85 10			CONTIN:	STA	POKMSK	; STORE NEW VALUE TO SYSTEM MASK

2332	EC33	BD 0E	D2				STA	IRQEN		; STORE TO ACTUAL REGISTER

2333					;

2334					;

2335	EC36	A9 28					LDA	#$28		; CLOCK CH.3 WITH 1.79 MHZ

2336	EC38	8D 08	D2				STA	AUDCTL	; CLOCK CH.4 WITH CH. 3

2337					;

2338	EC3B	A2 06					LDX	#6		; SET PURE TONES. NO VOLUME

2339	EC3D	A9 A8					LDA	#$A8

2340	EC3F	A4 41					LDY	SOUNDR	; TEST QUIET I/0 FLAG

2341	EC41	D0 02					BNE	NOISE1	; NE IS NORMAL (NOISY)

2342	EC43	A9 A0					LDA	#$A0

2343	EC45	9D 01	D2		NOISE1:	STA	AUDC1,X

2344	EC48	CA					DEX

2345	EC49	CA					DEX

2346	EC4A	10 F9					BPL	NOISE1

2347					;

2348	EC4C	A9 A0					LDA	#$A0

2349	EC4E	8D 05	D2				STA	AUDC3		; TURN OFF SOUND ON CHANNEL 3

2350	EC51	AC 00	03				LDY	DDEVIC

2351	EC54	C0 60					CPY	#CASET

2352	EC56	F0 06					BEQ	CAS31		; BRANCH IF CASSETTE IS DESIRED

2353	EC58	8D 01	D2				STA	AUDC1		; OTHERWISE TURN OFF CHANNELS 1 AND 2

2354	EC5B	8D 03	D2				STA	AUDC2

2355					;

2356					;

2357	EC5E	60			CAS31:	RTS			; RETURN

2358

2359

2360

2361

2362

2363					;

2364					;

2365					;

2366					;

2367					;

2368					; DISABLE SEND AND DISABLE RECEIVE SUBROUTINES

2369					;

2370	EC5F	EA			SENDDS:	NOP

2371	EC60	A9 C7			RECVDS:	LDA	#$C7		; MASK OFF SERIAL BUS INTERRUPTS

2372	EC62	25 10					AND	POKMSK

2373	EC64	85 10					STA	POKMSK	; STORE NEW VALUE TO SYSTEM MASK

2374	EC66	8D 0E	D2				STA	IRQEN		; STORE TO ACTUAL REGISTER

2375					;

2376	EC69	A2 06					LDX	#6

2377	EC6B	A9 00					LDA	#0

2378	EC6D	9D 01	D2		ZERIT:	STA	AUDC1,X

2379	EC70	CA					DEX

2380	EC71	CA					DEX

2381	EC72	10 F9					BPL	ZERIT		; TURN OFF AUDIO VOLUME

2382					;

2383	EC74	60					RTS	 		;RETURN

2384					;

2385					;

2386					;

2387					;

2388					;

2389					;

2390					;

2391					;

2392					;

2393					;

2394					; SET DDEVICE	TIME OUT VALUES IN Y,X SUBROUTINE

2395					;

2396	EC75	AD 06 03		STTMOT:	LDA	DTIMLO 	; GET DEVICE TIME OUT IN 1 SECOND INCR

2397	EC78	6A					ROR	A 		; PUT 6 HI BITS IN X. LO 2 BITS IN Y

2398	EC79	6A					ROR	A

2399	EC7A	A8					TAY			; TEMP SAVE

2400	EC7B	29 3F					AND	#$3F 		; MASK OFF 2 HI BITS

2401	EC7D	AA					TAX			; THIS IS HI BYTE OF TIME OUT

2402					;

2403	EC7E	98					TYA			; RESTORE

2404	EC7F	6A					ROR	A

2405	EC80	29 C0					AND	#$C0 		; MASK OFF ALL BUT 2 HI BITS

2406	EC82	A8					TAY			; THIS IS LO BYTE OF	TIME OUT

2407					;

2408	EC83	60					RTS

2409					;

2410					;

2411					;

2412					;

2413					;

2414					;

2415					;

2416					;

2417					;

2418					;

2419	EC84	0F EB			INTTBL:	.WORD	ISRSIR 	; SERIAL INPUT READY

2420	EC86	90 EA					.WORD	ISRODN 	; OUTPUT DATA NEEDED

2421	EC88	CF EA					.WORD	ISRTD 	; TRANSMISSION DONE

2422					;

2423	00EB				SIRHI		=	ISRSIR/256 	; SERIAL INPUT READY	ISR ADDRESS

2424	000F				SIRLO		=	(-256)*SIRHI+ISRSIR

2425	00EA				ODNHI		=	ISRODN/256 	; OUTPUT DATA NEEDED	ISR ADDRESS

2426	0090				ODNLO		=	(-256)*ODNHI+ISRODN

2427	00EA				TDHI		=	ISRTD/256 	; TRANSMISSION DONE I	SR ADDRESS

2428	00CF				TDLO		=	(-256)*TDHI+ISRTD

2429					;

2430					;

2431					;

2432					;

2433					; SEND A	DATA	FRAME TO AN INTELLIGENT PERIPHERAL	SUBROUTINE

2434					;

2435					;

2436	ECBA	A2 01			SENDIN:	LDX	4301

2437	ECBC	A0 FF			DELAY0:	LDY	#$FF

2438	ECBE	88			DELAY1:	DEY

2439	ECBF	D0 FD					BNE	DELAY1

2440	EC91	CA					DEX

2441	EC92	D0 FS					BNE	DELAY0

2442					;

2443	EC94	20 6B	EA				JSR	SEND		; GO SEND THE DATA FRAME

2444					;

2445	EC97	A0 02					LDY	#CTIMLO	; SET ACK TIME OUT

2446	EC99	A2 00					LDX	#CTIMHI

2447	EC9B	20 B9	ED		WAITER:	JSR	SETVBX

2448					;

2449	EC9E	20 1A	EA		JSR	WAIT	; WAIT FOR ACK

2450					;

2451	ECA1	98					TYA			; IF Y=0, A TIME OUT OR NACK OCCURED

2452					;

2453	ECA2	60					RTS			; RETURN

2454					;

2455					;

2456					;

2457					;

2458					;

2459					;

2460					;

2461					;

2462					;

2463					;

2464					;

2465					; COMPUTE VALUE FOR POKEY FREQ REGS FOR THE BAUD RATE AS

2466					; MEASURED BY AN INTERVAL OF THE 'VCOUNT' TIMER.

2467					;

2468	ECA3	8D 10	03		COMPUT:	STA	TIMER2

2469	ECA6	8C 11	03				STY	TIMER2+1	; SAVE FINAL TIMER VALUE

2470	ECA9	20 04	ED				JSR	ADJUST	; ADJUST VCOUNT VALUE

2471	ECAC	8D 10	03				STA	TIMER2	; SAVE ADJUSTED VALUE

2472	ECAF	AD 0C	03				LDA	TIMER1

2473	ECB2	20 04	ED				JSR	ADJUST	; ADJUST

2474	ECB5	8D 0C	03				STA	TIMER1	; SAVE ADJUSTED TIMERI VALUE

2475	ECBB	AD 10	03				LDA	TIMER2

2476	ECBB	38					SEC

2477	ECBC	ED 0C	03				SBC	TIMER1

2478	ECBF	8D 12	03				STA	TEMP1		; FIND VCOUNT DIFFERENCE

2479	ECC2	AD 11	03				LDA	TIMER2+1

2480	ECC5	38					SEC

2481	ECC6	ED 0D	03				SBC	TIMER1+1

2482	ECC9	AS					TAY			; FIND VBLANK COUNT DIFFERENCE

2483							.IF	PALFLG

2484							LDA	#-$9C

2485					HITIMR:	CLC

2486							ADC	#$9C

2487							.ENDIF

2488							.IF	PALFLG-1

2489	ECCA	A9 7D					LDA	#-$83

2490	ECCC	18			HITIMR:	CLC

2491	ECCD	69	83				ADC	#$83		; ACCUMULATE MULTIPLICATION

2492							.ENDIF

2493	ECCF	88					DEY

2494	ECD0	10 FA					BPL	HITIMR	; DONE?

2495	EC D2	18					CLC

2496	ECD3	6D 12	03				ADC	TEMPI		; TOTAL VCOUNT DIFFERENCE

2497	ECD6	A8			FINDX:	TAY			; SAVE ACCUM

2498	ECD7	4A					LSR	A

2499	ECDB	4A					LSR	A

2500	ECD9	4A					LSR	A

2501	ECDA	0A					ASL	A

2502	ECDB	38					SEC

2503	ECDC	E9 16					SBC	#22		; ADJUST TABLE INDEX

2504	ECDE	AA					TAX			; DIVIDE INTERVAL BY 4 TO GET TABLE INDEX

2505	ECDF	98					TYA			; RESTORE PCCUM

2506	ECE0	29 07					AND	#7

2507	ECE2	A8					TAY			; PULL OFF 3 LO BITS OF INTERVAL

2508	ECE3	A9 F5					LDA	#-11

2509	ECE5	18			DOINTP:	CLC

2510	ECE6	69 0B					ADC	#11		; ACCUMULATE INTERPOLATION CONSTANT

2511	ECE8	88					DEY

2512	ECE9	10 FA					BPL	DOINTP	; INTERPOLATION CONSTANT COMPUTATION DONE?

2513					;

2514	ECEB	A0 00			ENINTP:	LDY	#0

2515	ECED	8C 0E	03				STY	ADDCOR	; CLEAR ADDITION CORRECTION FLAG

2516	ECF0	38					SEC

2517	ECF1	E9 07					SBC	#7		; ADJUST INTERPOLATION CONSTANT

2518	ECF3	10 03					BPL	PLUS

2519	ECF5	CE 0E	03				DEC	ADDCOR

2520	ECF8	18			PLUS:		CLC

2521	ECF9	7D D0	ED				ADC	POKTAB,X	; ADD CONSTANT TO LO BYTE TABLE VALUE

2522	ECFC	A8					TAY			; LO BYTE POKEY FREG VALUE

2523	ECFD	AD 0E	03				LDA	ADDCOR

2524	ED00	7D D1	ED				ADC	POKTAB+1,X	; ADD CARRY TO HI BYTE TABLE VALUE

2525					; HI BYTE POKEY FREQ VALUE

2526	ED03	60					RTS

2527					;

2528					;

2529

2530					; ROUTINE TO ADJUST VCOUNT VALUE

2531					;

2532	ED04	C9 7C			ADJUST:	CMP	#$7C

2533	ED06	30 04					BMI	ADJ1		; LARGER THAN '7C' ?

2534	ED0B	38					SEC			; YES,

2535	ED09	E9 7C					SBC	#$7C

2536	ED0B	60					RTS

2537	ED0C	18			ADJ1:		CLC

2538							.IF	PALFLG

2539							ADC	#$20

2540							.ENDIF

2541							.IF	PALFLG-1

2542	ED0D	69 07					ADC	#$7

2543							.ENDIF

2544	ED0F	60					RTS

2545					;

2546					;

2547					;

2548					;

2549					;

;

2550					;

2551					;

2552					; INITIAL BAUD RATE MEASUREMENT -- USED TO SET THE

2553					; BAUD RATE	AT THE START OF A RECORD.

2554					;

2555					; IT IS ASSUMED THAT THE FIRST TWO BYTES OF EVERY

2556					; RECORD ARE 'AA' HEX.

2557					;

2558	ED10	A5 11			BEGIN:	LDA	BRKKEY

2559	ED12	D0 03					BNE	NTBRK2

2560	ED14	4C A0	ED				JMP	BROKE		; JUMP IF BREAK KEY PRESSED

2561					;

2562	ED17	78			NTBRK2:	SEI

2563					;

2564	ED18	AD 17	03				LDA	TIMFLG

2565	ED1B	D0 02					BNE	OKTIM1	; BRANCH IF NOT TIMED OUT

2566	ED1D	F0 25					BEQ	TOUT1		; BRANCH IF TIME OUT

2567

2568	ED1F	AD 0F	D2		OKTIM1:	LDA	SKSTAT

2569	ED22	29 10					AND	#$10		; READ SERIAL PORT

2570	ED24	D0 EA					BNE	BEGIN		; START BIT?

2571	ED26	SD 16	03				STA	SAVIO		; SAVE SER. DATA IN

2572	ED29	AE 0B	D4				LDX	VCOUNT	; READ VERTICAL LINE COUNTER

2573	ED2C	A4 14					LDY	RTCLOK+2	; READ LO BYTE OF VBLANK CLOCK

2574	ED2E	8E 0C	03				STX	TIMERI

2575	ED31	8C 0D	03				STY	TIMER1+1	; SAVE INITIAL TIMER VALUE

2576					;

2577	ED34	A2 01					LDX	#1		; SET MODE FLAG

2578	ED36	8E 15	03				STX	TEMP3

2579	ED39	A0 0A					LDY	#10		; SET BIT COUNTER FOR 10 BITS

2580	ED3B	A5 11			COUNT:	LDA	BRKKEY

2581	ED3D	F0 61					BEQ	BROKE		; BRANCH IF BREAK KEY PRESSED

2582					;

2583	ED3F	AD 17	03				LDA	TIMFLG

2584	ED42	D0 04					BNE	OKTIMR	; BRANCH IF NOT TIMED OUT

2585	ED44	58			TOUT1:	CLI

2586	ED45	4C 0A	EB				JMP	TOUT		; BRANCH IF TIME OUT

2587

2588	ED48	AD 0F	D2		OKTIMR:	LDA	SKSTAT

2589	ED4B	29 10					AND	#$10		; READ SERIAL PORT

2590	ED4D	CD 16	03				CMP	SAVIO		; DATA IN CHANGED YET?

2591	ED50	F0 E9					BEQ	COUNT

2592	ED52	8D 16	03				STA	SAVIO		; YES SAVE SER. DATA IN

2593	ED55	88 					DEY			; DECR. BIT COUNTER

2594	ED56	D0 E3					BNE	COUNT		; DONE?

2595					;

2596	ED58	CE 15	03				DEC	TEMP3		; YES

2597	ED5B	30 12					BMI	GOREAD	; DONE WITH BOTH MODES?

2598	ED5D	AD 0B	D4				LDA	VCOUNT

2599	ED60	A4 14					LDY	RTCLOK+2	; READ TIMER LO & HI BYTES

2600	ED62	20 A3	EC				JSR	COMPUT	; NO. COMPUTE BAUD RATE

2601	ED65	8C EE	02				STY	CBAUDL

2602	ED68	8D EF	02				STA	CBAUDH	; SET BAUD RATE INTO RAM CELLS

2603	ED6B	A0 09					LDY	#9		; SET BIT COUNTER FOR 9 BITS

2604	ED6D	D0 CC					BNE	COUNT

2605					;

2606	ED6F	AD EE	02		GOREAD:	LDA	CBAUDL

2607	ED72	8D 04	D2				STA	AUDF3

2608	ED75	AD EF	02				LDA	CBAUDH

2609	ED78	SD 06	D2				STA	AUDF4		; SET POKEY FREQ BEGS FOR BAUD RATE

2610	ED7B	A9 00					LDA	#0

2611	ED7D	8D 0F	D2				STA	SKSTAT

2612	ED80	AD 32	02				LDA	SSKCTL

2613	ED83	8D 0F	D2				STA	SKSTAT	; INIT. POKEY SERIAL PORT

2614	ED86	A9 55					LDA	#$55

2615	EDB8	91 32					STA	(BUFRLO),Y	; STORE '$55' AS FIRST RCV. BUFFER

2616	ED8A	C8					INY

2617	ED8B	91 32					STA	(BUFRLO),Y

2618	ED8D	A9 AA					LDA	#$AA

2619	EDBF	85 31					STA	CHKSUM	; STORE CHECKSUM FOR 2 BYTES OF '$AA'

2620	ED91	18					CLC

2621	ED92	A5 32					LDA	BUFRLO

2622	ED94	69 02					ADC	#2

2623	ED96	85 32					STA	BUFRLO

2624	ED98	A5 33					LDA	BUFRHI

2625	ED9A	69 00					ADC	#0

2626	ED9C	85 33					STA	BUFRHI	; INCR. BUFFER POINTER BY 1

2627	ED9E	58					CLI

2628	ED9F	60					RTS

2629					;

2630					;

2631					;

2632	EDA0	20 5F	EC		BROKE:	JSR	SENDDS	; BREAK KEY WAS PRESSED. SO PREPARE

2633	EDA3	A9 3C					LDA	#MOTRST	; TO RETURN

2634	EDA5	8D 02	D3				STA	PACTL	; TURN OFF MOTOR

2635	EDAB	8D 03	D3				STA	PBCTL	; RAISE NOT COMMAND LINE

2636					;

2637	EDAB	A9 80					LDA	#BRKABT

2638	EDAD	85 30					STA	STATUS	; STORE BREAK ABORT STATUS CODE

2639					;

2640	EDAF	AE 18	03				LDX	STACKP

2641	EDB2	9A					TXS			; RESTORE STACK POINTER

2642					;

2643	EDB3	C6 11					DEC	BRKKEY	; SET BREAK KEY FLAG TO NONZERO

2644	EDB5	58					CLI			; ALLOW IRQ'S

2645					;

2646	ED86	4C 0D	EA				JMP	RETURN	; GO RETURN

2647					;

2648					;

2649					;

2650					;

2651					;

2652	EDB9	A9 EC			SETVBX:	LDA	#JTADRL	; STORE TIME OUT ROUTINE ADDRESS

2653	EDBB	8D 26	02				STA 	CDTMA1

2654	EDBE	A9 EB					LDA 	#JTADRH

2655	EDC0	8D 27	02				STA 	CDTMA1+1

2656

I

2657	EDC3	A9 01					LDA 	#1 		; SET FOR TIMER 1

2658

2659	EDC5	78					SEI 			; THE SETVBL ROUTINE NEEDS T	HIS TO CUT SHORT

2660	EDC6	20 5C	E4				JSR  	SETVBV 	; ANY VBLANKS THAT OCCUR

2661	EDC9	A9 01					LDA 	#1 		; SET FOR TIMER 1

2662	EDCB	8D 17	03				STA 	TIMFLG 	; SET FLAG TO NOT TIMED OUT

2663	EDCE	58					CLI

2664	EDCF	60					RTS

2665					;

2666					;

2667					;

2668					;

2669					;

2670					;

2671					;

2672					; 'VCOUNT' INTERVAL TIMER MEASUREMENT -- TO -- POKEY FR	EQ REG VALUE

2673					; CONVERSION TABLE

2674					;

2675					;

2676					; THE VALUES STORED IN THE TABLE ARE 'AUDF+7 '.

2677					;

2678					; THE FOLLOWING FORMULAS WERE USED TO DETERMINE THE TABLE VALUES:

2679					;

2680					; F OUT= F IN/(2*(AUDF+M)) ~ WHERE F IN=1	.78979 MHZ. & M=7

2681					;

2682					; FROM THIS WAS DERIVED THE FORMULA USED TO COMP	UTE THE

2683					; TABLE VALUES BASED ON A MEASUREMENT OF THE PER	IOD BY

2684					; AN INTERVAL OF THE 'VCOUNT' TIMER.

2685					;

2686					; AUDF+7=(11.365167)*T OUT WHERE T OUT=#	OF COUNTS

2687					; (127 USEC.RESOLUTION) OF 'VCOUNT' FOR 1

2688					; CHARACTER TIME (10 BIT TIMES).

2689					;

2690					;

2691					;

2692					;

2693					;

2694					;	 		AUDF+7	BAUD RATE	VCOUNT INTERVAL

2695					;	.WORD 	$27C 		; 1407		56

2696					;	.WORD 	$2D7 		; 1231		64

2697					;	.WORD 	$332 		; 1094		72

2698					;	.WORD 	$38D 		; 985			80

2699	EDD0	E8 03			POKTAB: .WORD 	$3E8 		; 895			88

2700	EDD2	43 04			 	.WORD 	$443 		; 820			96

2701	EDD4	9E 04			 	.WORD 	$49E 		; 757			104

2702	EDD6	F9 04			 	.WORD 	$4F9 		; 703			112

2703	EDDB	54 05			 	.WORD 	$554 		; 656			120

2704	EDDA	AF 05			 	.WORD 	$5AF 		; 615			128

2705	EDDC	0A 06			 	.WORD 	$60A 		; 579			136

2706	EDDE	65 06			 	.WORD 	$665 		; 547			144

2707	EDE0	C0 06			  	.WORD		$6C0		; 518			152

2708	EDE2	IA 07			  	.WORD		$71A		; 492			160

2709	EDE4	75 07			  	.WORD		$775		; 469			168

2710	EDE6	D0 07			  	.WORD		$7D0		; 447			176

2711					  	.WORD		$82B		; 428			184

2712					  	.WORD		$886		; 410			192

2713					  	.WORD		$8E1		; 394			200

2714					  	.WORD		$93C		; 379			208

2715					  	.WORD		$997		; 365			216

2716				 	  	.WORD		$9F2		; 352			224

2717					  	.WORD		$A4D		; 339			232

2718					  	.WORD		$AA8		; 328			240

2719					  	.WORD		$B03		; 318			248

2720					;

2721					;

2722					;

2723					;

2724					; *********************************************************

2725	EDEB					CRNTP3 =*

2726							*=$14

2727	0014	02			SIOSPR: .BYTE	DSKORG-CRNTP3	; ^GSIOL IS TOO LONG

2728					;

2729					.TITLE 'DISK ***** DISKP.SRC ***** 3/9/79 ***** 4:00:00 P. M.'

2730					;

2731					;

2732					;

2733					;

2734					;

2735					;

2736	0002				STATVH	= 	DVSTAT/256

2737	00EA				STATVL	= 	(-256)*STATVH+DVSTAT ;STATUS	POINTER

2738					;

2739					;

2740					;

2741					;

2742					;	CONSTANT EQUATES

2743					;

2744	0031				DISKID	= 	$31 		; SERIAL BUS DISK	I. D.

2745	0050				PUTSEC	=	$50 		; DISK PUT SECTOR	DCB COMMAND

2746					; READ	= 	452 		; DISK GET SECTOR	DCB COMMAND

2747					; WRITE	= 	457 		; DISK PUT SECTOR	WITH READ CHECK DCB COMMAND

2748	0053				STATC		=	$53 		; DISK STATUS DCB	COMMAND

2749	0021				FOMAT		=	$21 		; DISK FORMAT DCB	COMMAND !!!!! *****

2750	0000				NODAT		=	0 		; SIO COMMAND FOR	"NO DATA" OPERATION

2751	0040				GETDAT	= 	$40 		; SIO COMMAND FOR	"DATA FROM DEVICE"

2752	0080				PUTDAT	= 	$80 		; SIO COMMAND FOR	"DATA TO DEVICE"

2753					;

2754					;

2755					;	 VECTORS

2756					;

2757							*=$E450

2758					;

2759	E450 4C EA ED				JMP 	DINIT 		; DISK INIT. VECTOR

2760	E453 4C F0 ED				JMP 	DSKIF 		; DISK INTERFACE ENTRY POINT

2761					;

2762					;

2763					;

2764					;

2765					;

2766					;

2767					;	CONSTANTS

2768					;

2769							*=DSKORG

2770					;

2771					;

2772					;

2773					;

2774					;

2775					;

2776					;

2777					;

2778					;

2779					; ***********************************************************************

2780					;			DISK INTERFACE ROUTINE STARTS HERE

2781					; ***********************************************************************

2782					;

2783					;

2784					;

2785					;

2786					; 	DISK	INTERFACE	INITIALIZATION ROUTINE

2787					;

2788	EDEA	A9 A0			DINIT:	LDA	#160

2789	EDEC	8D 46	02				STA	DSKTIM	; SET INITIAL DISK TIMEOUT TO 160 SEC

2790	EDEF	60					RTS

2791					;

2792					;

2793					;

2794					;	DISK	INTERFACE	ENTRY POINT

2795					;

2796	EDF0	A9 31			DSKIF:	LDA	#DISKID

2797	EDF2	8D 00	03				STA	DDEVIC	; SET SERIAL BUS I.D IN DCB

2798	EDF5	AD 46	02				LDA	DSKTIM

2799	EDFB	AE 02	03				LDX	DCOMND

2800	EDFB	E0 21					CPX	#FOMAT	; IS COMMAND A FORMAT COMMAND?

2801	EDFD	F0 02					BEQ	PUTDTO

2802	EDFF	A9 07					LDA	#7		; NO, SET TIMEOUT TO 7 SECS.

2803	EE01	8D 06	03		PUTDTO:	STA	DTIMLO	; PUT DISK TIMEOUT IN DCB

2804	EE04	A2 40					LDX	#GETDAT	; SET "GET DATA" COMMAND FOR SIO

2805	EE06	A0 80					LDY	#$80		; SET BYTE COUNT TO 128

2806	EE08	AD 02	03				LDA	DCOMND	; READ COMMAND IN DCB

2807	EE0B	C9 57					CMP	#WRITE	; IS COMMAND A "PUT SECTOR" COMMAND?

2808	EE0D	D0 02					BNE	CKSTC

2809	EE0F	A2 80					LDX	#PUTDAT	; YES. SET "PUT DATA" COMMAND FOR SIO

2810	EE11	C9 53			CKSTC:	CMP	#STATC	; IS COMMAND A STATUS COMMAND?

2811	EE13	D0 0C					BNE	PUTCNT

2812	EE15	A9 EA					LDA	#STATVL

2813	EE17	8D 04	03				STA	DBUFLO

2814	EE1A	A9 02					LDA	#STATVH

2815	EElC	8D 05	03				STA	DBUFHI	; SET BUFFER ADDR TO GLOBAL STATUS BUFFER

2816	EE1F	A0 04					LDY	#4		; YES. SET BYTE COUNT TO 4

2817	EE21	8E 03	03		PUTCNT:	STX	DSTATS	; PUT STATUS COMMAND FOR SIO IN DCB

2818	EE24	8C 08	03				STY	DBYTLO

2819	EE27	A9 00					LDA	#0

2820	EE29	8D 09	03				STA	DBYTHI	; PUT BYTE COUNT IN DCB

2821	EE2C	20 59	E4				JSR	SIOV		; CALL SERIAL I/0.

2822	EE2F	10 01					BPL	GOODST	; NO ERROR

2823	EE31	60					RTS			; NO, GO BACK

2824	EE32	AD 02	03	GOODST:	LDA	DCOMND		; READ THE COMMAND

2825	EE35	C9 53					CMP	#STATC	; WAS IT A STATUS COMMAND?

2826	EE37	D0 0A					BNE	PUTBC

2827	EE39	20 6D	EE				JSR	PUTADR	; PUT BUFFER ADDR IN TEMP REG.

2828	EE3C	A0 02					LDY	#2

2829	EE3E	B1 15					LDA	(BUFADR),Y 	; READ DISK TIMEOUT VALUE BYTE OF STATUS

2830	EE40	8D 46	02				STA	DSKTIM	; PUT IT IN DISK TIMEOUT REG.

2831	EE43	AD 02	03		PUTBC:	LDA	DCOMND

2832	EE46	C9 21					CMP	#FOMAT	; WAS COMMAND A FORMAT COMMAND?

2833	EE48	D0 1F					BNE	ENDDIF

2834	EE4A	20 6D	EE		FMTD:		JSR	PUTADR	; YES PUT BUFFER ADDR INTO TEMP REG

2835	EE4D	A0 FE					LDY	#$FE		; SET BUFFER POINTER

2836	EE4F	CS			TWICE:	INY

2837	EE50	CS					INY			; INCR BUFFER POINTER BY 2

2838	EE51	B1 15			RDBAD:	LDA	(BUFADR),Y	; READ LO BYTE BAD SECTOR DATA

2839	EE53	C9 FF					CMP	#$FF

2840	EE55	D0 F8					BNE	TWICE		; IS IT "FF" ?

2841	EE57	CS					INY			; YES,

2842	EE58	B1 15					LDA	(BUFADR),Y	; READ HI BYTE BAD SECTOR DATA

2843	EE5A	CS					INY

2844	EE5B	C9 FF					CMP	#$FF

2845	EE51)	D0 F2					BNE	RDBAD		; IS IT "FF" ?

2846	EE5F	88					DEY

2847	EE60	88					DEY			; YES,

2848	EE61	8C 08	03				STY	DBYTLO	; PUT BAD SECTOR BYTE COUNT INTO DCB

2849	EE64	A9 00					LDA	#0

2850	EE66	8D 09	03				STA	DBYTHI

2851	EE69	AC 03	03		ENDDIF:	LDY	DSTATS

2852	EE6C	60					RTS

2853					;

2854					;

2855					;

2856					;

2857					; 	S U B	R O U T I N E S

2858					;

2859					;

2860					; 	PUT BUFFER ADDR FROM	DCB INTO TEMP REG

2861					;

2862	EE6D	AD 04	03		PUTADR:	LDA	DBUFLO

2863	EE70	85 15					STA	BUFADR

2864	EE72	AD 05	03				LDA	DBUFHI

2865	EE75	85 16					STA	BUFADR+1	; PUT BUFFER ADDR IN TEMP REG

2866	EE77	60					RTS

2867					; ****************************************************************

2868					;

2869					;

2870					; 	SPARE	BYTE OR MODULE	TOO LONG FLAG

2871					;

;

2872	EE78				CRNTP4	=	*

2873					;

2874							*=$14

2875	0014	00			DSKSPR:	.BYTE	PRNORG-CRNTP4 	; ^GDISKP TOO LONG

2876					;

2877					; .PAGE

2878					.TITLE	'PRINTER	***** PRINTP.SRC ***** 3/9/79 ***** 4:00:00 P

2879					;

2880					;

2881					;

2882					;

2883					;

2884					;

2885					;

2886					;

2887					;

2888					;

2889					;

2890					; 	DEVICE NUMBER OR CODE EQUATES

2891					;

2892	0002				OPNOUT		$2		; IOCB OPEN FOR OUTPUT COMMAND

2893	0028				NBUFSZ	=	40		; PRINT NORMAL BUFFER SIZE

2894	0014				DBUFSZ	=	20		; PRINT DOUBLE BUFFFER SIZE

2895	001D				SBUFSZ	=	29		; PRINT SIDEWAYS BUFFER SIZE

2896	0040				PDEVN		=	$40		; PRINTER DEVICE NUMBER

2897					; STATC	=	$53		; DCB STATUS COMMAND CODE

2898	0057				WRITEC	=	$57		; DCB WRITE COMMAND

2899	0020				SPACE		=	$20		; ASCII SPACE CHAR.

2900	004E				N		=	$4E		; ASCII "N" CHAR.

2901	0044				D		=	$44		; ASCII "D" CHAR.

2902	0053				S		=	$53		; ASCII "S" CHAR.

2903					;

2904					;

2905					; 	PRINTER	HANDLER ENTRY POINTS

2906					;

2907					;

2908					;

2909					;

2910							*=$E430

2911					;

2912	E430	9E EE			.WORD		PHOPEN-1		; PRINTER HANDLER OPEN

2913	E432	DB EE			.WORD		PHCLOS-1		; PH CLOSE

2914	E434	9D EE			.WORD		BADST-1		; PH READ

2915	E436	A6 EE			.WORD		PHWRIT-1		; PH WRITE

2916	E438	80 EE			.WORD		PHSTAT-1		; PH STATUS

2917	E43A	9D EE			.WORD		BADST-1		; PH SPECIAL

2918	E43C	4C 78 EE				JMP	PHINIT	; PH INIT.

2919	E43F	00			.BYTE	0				; ROM FILLER

2920					;

2921					;

2922					;

2923					;

2924					;

2925							*=PRNORG

2926					;

2927					;

2928					;

2929					;

2930					; PRINTER	HANDLER INITIALIZATION ROUTINE

2931					;

2932	EE78	A9 lE			PHINIT:	LDA	#30

2933	EE7A	85 1C					STA	PTIMOT	; SET UP INITIAL PRINTER TIMEOUT OF 30 SEC.

2934	EE7C	60					RTS

2935					;

2936					;

2937					;	PRINTER HANDLER CONSTANTS

2938					;

2939	EE7D	EA 02			PHSTLO:	.WORD	DVSTAT	; STATUS BUFFER POINTER

2940	EE7F	C0 03			PHCHLO:	.WORD	PRNBUF	; CHAR. BUFFER POINTER

2941					;

2942					;

2943					;

2944					;	********************************

2945					;	PRINTER	HANDLER	ROUTINES

2946					;	********************************

2947					;

2948					;

2949					;

2950					;

2951					;

2952					;	PRINTER HANDLER STATUS ROUTINE

2953					;

2954	EE81	A9 04			PHSTAT:	LDA	114

2955	EE83	85 lE					STA	PBUFSZ	; SET BUFFER SIZE TO 4 BYTES

2956	EE85	AE 7D	EE				LDX	PHSTLO

2957	EE88	AC 7E	EE				LDY	PHSTLO+1 	; SET POINTER TO STATUS BUFFER

2958	EEBB	A9 53					LDA	#STATC	;SET COMMAND TO "STATUS"

2959	EEBD	8D 02	03				STA	DCOMND	;SET STATUS COMMAND

2960	EE90	8D 0A	03				STA	DAUX1

2961	EE93	20 E6	EE				JSR	SETDCB	; GO SETUP DCB

2962	EE96	20 59	E4				JSR	SIOV		; SEND STATUS COMMAND

2963	EE99	30 03					BMI	BADST		; GO IF ERROR

2964	EE9B	20 14	EF				JSR	PHPUT		; YES. PUT STATUS INTO GLOBAL BUFFER.

2965	EE9E	60			BADST:	RTS

2966					;

2967					;

2968					;

2969					;

2970					;	PRINTER	HANDLER	OPEN ROUTINE

2971					;

2972	EE9F	20 81	EE		PHOPEN:	JSR	PHSTAT	; DO STATUS COMMAND TO SIO

2973	EEA2	A9 00					LDA	#0

2974	EEA4	85 1D					STA	PBPNT		; CLEAR PRINT BUFFER POINTER

2975	EEA6	60					RTS

2976					;

2977					;

2978					;

2979					;

2980					;	PRINTER	HANDLER	WRITE ROUTINE

2981					;

2982	EEA7	85 1F			PHWRIT:	STA	PTEMP		; SAVE ACCUM

2983	EEA9	20 1A	EF				JSR	PRMODE	; GO DETERMINE PRINT MODE

2984	EEAC	A6 1D					LDX	PBPNT

2985	EEAE	A5 1F					LDA	PTEMP		; GET CHAR. SENT BY CIO

2986	EEB0	9D C0	03				STA	PRNBUF,X	; PUT CHAR. IN PRINT BUFFER

2987	EEB3	EB 					INX			; INCR. BUFFER POINTER

2988	EEB4	E4 1E					CPX	PBUFSZ	; BUFFER POINTER=BUFFER SIZE?

2989	EEB6	F0 13					BEQ	BUFFUL

2990	EEBB	86 1D					STX	PBPNT		; SAVE BUFFER POINTER

2991	EEBA	C9 9B					CMP	#CR		; IS CHAR. = EOL ?

2992	EEBC	F0 03					BEQ	BLFILL	; IF YES, GO DO BLANK FILL.

2993	EEBE	A0 01					LDY	#SUCCES	; PUT GOOD STATUS IN Y REG FOR CIO.

2994	EEC0	60					RTS

2995	EEC1	A9 20			BLFILL:	LDA	#SPACE	; PUT BLANK IN ACCUM.

2996	EEC3	9D C0	03		FILLBF:	STA	PRNBUF,X	; STORE IT IN PRINT BUFFER.

2997	EEC6	E8					INX

2998	EEC7	E4 lE					CPX	PBUFSZ

2999	EEC9	D0 F8					BNE	FILLBF	; BUFFER BLANK FILLED?

3000	EECB	A9 00			BUFFUL:	LDA	#0

3001	EECD	85 ID					STA	PBPNT		; CLEAR PRINT BUFFER POINTER

3002	EECF	AE 7F	EE				LDX	PHCHLO

3003	EED2	AC 80	EE				LDY	PHCHLO+1	; SET POINTER TO PRINT BUFFER

3004	EED5	20 E6	EE				JSR	SETDCB	; GO SETUP DCB

3005	EEDB	20 59	E4				JSR	SIOV		; SEND PRINT COMMAND

3006	EEDB	60					RTS			; YES.

3007					;

3008					;

3009					;

3010					;

3011					; 	PRINTER	HANDLER CLOSE ROUTINE

3012					;

3013	EEDC	20 1A	EF		PHCLOS:	JSR	PRMODE	; GO DETERMINE PRINT MODE

3014	EEDF	A6 1D					LDX	PBPNT

3015	EEE1	D0 DE					BNE	BLFILL

3016	EEE3	A0 01					LDY	#SUCCES

3017	EEE5	60					RTS

3018					;

3019					;

3020					;

3021					;

3022					;

3023					;

3024					;

3025					;

3026					;	S U B R O U T I N E S

3027					;

3028					;

3029					;

3030					;

3031					;

3032					; SET UP DCB TO CALL SIO

3033					;

3034	EEE6	BE 04	03		SETDCB:	STX	DBUFLO

3035	EEE9	BC 05	03				STY	DBUFHI	; SET BUFFER POINTER

3036	EEEC	A9 40					LDA	#PDEVN

3037	EEEE	BD 00	03				STA	DDEVIC	; SET PRINTER BUS I. D. FOR DCB

3038	EEF1	A9 01					LDA	#1

3039	EEF3	8D 01	03				STA	DUNIT		; SET UNIT NUMBER TO 1

3040	EEF6	A9 80					LDA	#$80		; DEVICE WILL EXPECT DATA

3041	EEF8	AE 02	03				LDX	DCOMND

3042	EEFB	E0 53					CPX	#STATC	; STATUS COMMAND?

3043	EEFD	D0 02					BNE	PSIOC

3044	EEFF	A9 40					LDA	#$40		; EXPECT DATA FROM DEVICE

3045	EF01	8D 03	03		PSIOC: 	STA	DSTATS	; SET SIO MODE COMMAND.

3046	EF04	A5 1E					LDA	PBUFSZ

3047	EF06	8D 08	03				STA	DBYTLO	; SET LO BYTE COUNT

3048	EF09	A9 00					LDA	#0

3049	EF0B	8D 09	03				STA	DBYTHI	; SET HI BYTE COUNT

3050	EF0E	A5 1C					LDA	PTIMOT

3051	EF10	SD 06	03				STA	DTIMLO	; SET DEVICE TIMEOUT COUNT

3052	EF13	60					RTS

3053					;

3054					;

3055					;

3056					;

3057					; GET DEVICE TIMEOUT FROM STATUS & SAVE IT

3058					;

;

3059	EF14	AD EC	02		PHPUT: 	LDA	DVSTAT+2

3060	EF17	85 1C					STA	PTIMOT	; SAVE DEVICE TIMEOUT

3061	EF19	60					RTS

3062					;

;

3063					;

3064					;

3065					;

3066					; DETERMINE PRINT MODE & SETUP PRINT BUFFER SIZE, DCB PRINT

3067					; 	COMMAND & DCB AUXI FOR PRINT MODE

3068					;

3069	EF1A	A0 57			PRMODE: 	LDY	#WRITEC	; PUT WRITE COMMAND IN Y REG

3070	EF1C	A5 2B					LDA	ICAX2Z	; READ PRINT MODE

3071	EF1E	C9 4E			CMODE: 	CMP	#N

3072	EF20	D0 04					BNE	CDUBL		; PRINT NORMAL ?

3073	EF22	A2 28					LDX	#NBUFSZ	; YES SET NORMAL CHAR. BUFFER SIZE

3074	EF24	D0 0E					BNE	SETBSZ

3075	EF26	C9 44			CDUBL: 	CMP	#D

3076	EF28	D0 04					BNE	CSIDE		; PRINT DOUBLE?

3077	EF2A	A2 14					LDX	#DBUFSZ	; YES SET DOUBLE CHAR. BUFFER SIZE

3078	EF2C	D0 06					BNE	SETBSZ

3079	EF2E	C9 53			CSIDE: 	CMP	#S		; PRINT SIDEWAYS ?

3080	EF30	D0 0B					BNE	GOERR		; IF NOT GO TO ERROR ROUTINE

3081	EF32	A2 1D					LDX	#SBUFSZ	; YES SET SIDEWAYS BUFFER SIZE

3082	EF34	86 1E			SETBSZ: 	STX	PBUFSZ	; STORE PRINT BUFFER SIZE

3083	EF36	8C 02	03				STY	DCOMND	; STORE DCB COMMAND

3084	EF39	8D 0A	03				STA	DAUX1		; STORE DCB AUXI PRINT MODE

3085	EF3C	60					RTS

3086	EF3D	A9 4E			GOERR: 	LDA	#N		; SET DEFAULT PRINT MODE TO NORMAL

3087	EF3F	D0 DD					BNE	CMODE

3088					; ************************************************************

3089					;

3090					;

3091					; SPARE BYTE OR MODULE TOO LONG FLAG

3092					;

3093  EF41 				CRNTP5	=	*

3094  3095 						*=$14

3096					;

3097  0014 00 			PRNSPR: .BYTE CASORG-CRNTP5 :^GPRINTP TOO LONG

3098					;

3099							.PAGE

3100							.TITLE 'CASSET HANDLER 3/12 (DKI:CASCV)'

3101	0003				CBUFH		= 	CASBUF/256

3102	00FD				CBUFL		= 	(-256)*CBUFH+CASBUF

3103	0040				SRSTA		= 	$40 		; SIO READ STATUS

3104	0080				SWSTA		= 	$80 		; SIO WRITE STATUS

3105					; MOTRGO	= 	$34

3106					; MOTRST	= 	$3C

3107					;

3108					;

3109	00FC				DTA		= 	$FC 		; DATA RECORD TYPE BYTE

3110	00FA				DTI		= 	$FA 		; LAST DATA RECORD

3111	00FE				EOT		=	$FE 		; END OF TAPE

3112	00FB				HDR		=	$FB 		; HEADER

3113	0002				TONE1		=	2 		; CHANGE TO RECORD MODE TONE

3114	0001				TONE2		=	1 		; PRESS PLAY TONE

3115					;

3116					;

3117					;

3118							*=CASETV

3119	E440	48 EF	2A F0		.WORD OPENC-1.CLOSEC-1.GBYTE-1,PBYTE-1,STATU-1, SPECIAL-1

3120	E444	D5 EF	0F F0		;

3121	E448	27 F0	4A EF		;

3122	E44C	4C 41	EF				JMP INIT

3123	E44F	00					.BYTE 0 ; ROM FILLER BYTE

3124					;

3125					;

3126					;

3127					; USED IN MONITP FOR CASSETTE BOOT

3128					;

3129							*=RBLOKV

3130	E47A	4C E9	EF				JMP 	RBLOK

3131					;

3132							*=CSOPIV

3133	E47D	4C 5D	EF				JMP 	OPINP

3134					;

3135					;

3136							*=CASORG

3137					;

3138					;

;

3139					; INIT ROUTINE

3140					;

3141	EF41	A9 CC			INIT:		LDA 	#$CC

3142	EF43	8D EE	02				STA 	CBAUDL

3143	EF46	A9 05					LDA 	#$05

3144	EF48	8D EF	02				STA 	CBAUDH 	; SET CASSET BAUD RATE TO 600

3145					SPECIAL:				; THATS ALL FOLKS

3146	EF4B	60					RTS

3147					.PAGE

3148					;

3149					; OPEN FUNCTION - WITH NO TIMING ADJUST

3150					;

3151	EF4C	A5 2B			OPENC:	LDA	ICAX2Z	; GET AX2

3152	EF4E	85 3E					STA	FTYPE		; SAVE IT FOR FUTURE REFERENCE

3153	EF50	A5 2A					LDA	ICAXIZ

3154	EF52	29 0C					AND	#$0C		; IN AND OUT BITS

3155	EF54	C9 04					CMP	#$04

3156	EF56	F0 05					BEQ	OPINP

3157	EF58	C9 OS					CMP	#$08		; SEE IF OPEN FOR OUTPUT

3158	EF5A	F0 39					BEQ	OPOUT

3159	EF5C	60					RTS			; IF ALREADY OPEN. RETURN LEAVING STATUS=f84

3160	EF5D	A9 00			OPINP:	LDA	#0

3161	EF5F	8D 89	02				STA	WMODE		; SET READ MODE

3162	EF62	85 3F					STA	FEOF		; NO EOF YET

3163	EF64	A9 01			SFH:		LDA	#TONE2	; TONE FOR PRESS PLAY

3164	EF66	20 58	F0				JSR	BEEP		; GO BEEP

3165	EF69	30 24					BMI	OPNRTN	; IF ERROR DURING BEEP

3166	EF6B	A9 34					LDA	#MOTRGO

3167	EF6D	8D 02	D3				STA	PACTL		; TURN MOTOR ON

3168					.IF	PALFLG

3169							LDY	#$E0

3170							LDX	#1

3171					.ENDIF

3172					.IF	PALFLG-1

3173	EF70	A0 40					LDY	#$40		; 5-31-79 9 SEC READ LEADER

3174	EF72	A2 02					LDX	#2

3175					.ENDIF

3176	EF74	A9 03					LDA	#3

3177	EF76	8D 2A	02				STA	CDTMF3

3178	EF79	20 5C	E4				JSR	SETVBV	; SET UP VBLANK TIMER

3179	EF7C	AD 2A	02		WAITTM:	LDA	CDTMF3

3180	EF7F	D0 FB					BNE	WAITTM	; WAIT FOR MOTOR TO COME UP TO SPEED

3181	EFB1	A9 80					LDA	#$80		; NEXT BYTE=NO BYTES IN BUFFER

3182	EF83	85 3D					STA	BPTR

3183	EF85	8D 8A	02				STA	BLIM

3184	EF88	4C D3	EF				JMP	OPOK		; OPEN OK

3185					;

3186					; OPEN FOR OUTPUT

3187					;

3188	EF8B	A0 80			PBRK:		LDY	#BRKABT	; BREAK KEY ABORT STATUS

3189	EF8D	C6 11					DEC	BRKKEY	; RESET BREAK KEY

3190	EFBF	A9 00			OPNRTN:	LDA	#0		; CLEAR WRITE MODE FLAG

3191	EF91	8D 89	02				STA	WMODE

3192	EF94	60					RTS			; AND EXIT.

3193					;

3194	EF95	A9 80			OPOUT:	LDA	#$80

3195	EF97	8D 89	02				STA	WMODE		; SET WRITE MODE

3196	EF9A	A9 02					LDA	#TONE1	; TELL USER TO TURN ON RECORD MODE

3197	EF9C	20 58	F0				JSR	BEEP

3198	EF9F	30 EE					BMI	OPNRTN	; IF ERROR DURING BEEP

3199	EFAl	A9 CC					LDA	#$CC		; SET BAUD RATE

3200	EFA3	8D 04 D2				STA	AUDF3		; WHICH SEEMS TO BE NESSECARY

3201	EFA6	A9 05					LDA	#$05		; FOR SOME OBSCURE REASON

3202	EFAB	8D 06	D2				STA	AUDF4

3203	EFAB	A9 60					LDA	#$60

3204	EFAD	8D 00	03				STA	DDEVIC

3205	EFB0	20 68	E4				JSR	SENDEV	; TELL POKEY TO WRITE MARKS

3206	EFB3	A9 34					LDA	#MOTRGO	; WRITE 5 SEC BLANK TAPE

3207	EFB5	8D 02	D3				STA	PACTL

3208	EFBB	A9 03					LDA	#3

3209					.IF	PALFLG

3210							LDX	#$3

3211							LDY	#$C0

3212					.ENDIF

3213					.IF	PALFLG-1

3214	EFBA	A2 04					LDX	#4		; 5/30/79 20 SEC LEADER

3215	EFBC	A0 80					LDY	#$80

3216					.ENDIF

3217	EFBE	20 5C	E4				JSR	SETVBV

3218	EFC1	A9 FF					LDA	#$FF

3219	EFC3	8D 2A	02				STA	CDTMF3

3220	EFC6	A5 11			WDLR: 	LDA	BRKKEY

3221	EFC8	F0 C1					BEQ	PERK		; IF BREAK DURING WRITE LEADER

3222	EFCA	AD 2A	02				LDA	CDTMF3

3223	EFCD	D0 F7					BNE	WDLR

3224	EFCF	A9 00					LDA	#0		; INIT BUFFER POINTER

3225	EFD1	85 3D					STA	BPTR

3226	EFD3	A0 01			OPOK: LDY	#SUCCES

3227	EFD5	60					RTS

3228						.PAGE

3229					;

3230					; GET	BYTE

3231					;

3232	EFD6	A5 3F			GBYTE:	LDA	FEOF		; IF AT EOF ALREADY

3233	EFDB	30 33					BMI	ISEOF		; RETURN EOF STATUS

3234	EFDA	A6 3D					LDX	BPTR		; BUFFER POINTER

3235	EFDC	EC SA	02				CPX	BLIM		; IF END OF BUFFER

3236	EFDF	F0 08					BEQ	RBLOK		; READ ANOTHER BLOCK

3237	EFE1	BD 00	04				LDA	CASBUF+3,X	; GET NEXT BYTE

3238	EFE4	E6 3D					INC	BPTR		;BUMP POINTER

3239	EFE6	A0 01					LDY	#SUCCES	; OK STATUS

3240	EFEB	60	 		GBX:		RTS

3241	EFE9	A9 52			RBLOK:	LDA	#'R		; READ OPCGDE

3242	EFEB	2095	F0				JSR	SIOSB		; SIO ON SYS BUF

3243	EFEE	98					TYA

3244	EFEF	30 F7					BMI	GBX		; IF SIO ERRORS. RETURN

3245	EFF1	A9 00					LDA	#0

3246	EFF3	85 3D					STA	BPTR		; RESET POINTER

3247	EFF5	A2 80					LDX	#$80		; DEFAULT # BYTES

3248	EFF7	AD FF	03				LDA	CASBUF+2

3249	EFFA	C9 FE					CMP	#EDT

3250	EFFC	F0 0D					BEQ	ATEOF		;IF HEADER, GO READ AGAIN

3251	EFFE	C9 FA					CMP	#DT1		; IF LAST DATA REC

3252	F000	D0 03					BNE	NLR

3253	F002	AE 7F	04				LDX	CASBUF+130	; LAST DATA RECORD, GET # BYTES

3254	F005	8E 8A	02		NLR:		STX	BLIM

3255	F008	4C D6	EF				JMP	GBYTE		; GET NEXT BYTE

3256	F00B	C6 3F			ATEOF:	DEC	FEOF		; SET FEOF

3257	F00D	A0 88			ISEOF:	LDY	#EOFERR	;ENDFILE STATUS

3258	F00F	60					RTS

3259						.PAGE

3260					;

3261					; PUT BYTE TO BUFFER

3262					;

3263	F010	A6 3D			PBYTE: 	LDX	BPTR		; BUFFER POINTER

3264	F012	9D 00 04				STA	CASBUF+3,X	; STORE CHAR AWAY

3265	F015	E6 3D					INC	BPTR		; BUMP POINTER

3266	F017	A0 01					LDY	#SUCCES	; OK STATUS

3267	F019	E0 7F					CPX	#127		; IF BUFFER FULL

3268	F01B	F0 01					BEQ	*+3

3269	F01D	60         				RTS

3270					; WRITE OUT THE	BUFFER

3271	F01E	A9 FC					LDA	#DTA		; RECORD TYPE = DATA

3272	F020	20 D2 F0				JSR	WSIOSB	; DO WRITE ON SYSTEM BUFFER

3273	F023	A9 00					LDA	#0

3274	F025	85 3D					STA	BPTR		; RESET BUFFER POINTER

3275	F027	60					RTS		; EXIT.

3276 						.PAGE

3277 					;

3278 					; STATUS - RETURN STATUS INFO THRU DVSTAT

3279					;

3280 F028 A0 01 			STATU: 	LDY #SUCCES

3281 F02A 60 					RTS

3282						.PAGE

3283					;

3284					; CLOSE

3285					;

3286	F02B	AD 89	02		CLOSEC:	LDA	WMODE		; SEE IF WRITING

3287	F02E	30 08					BMI	CLWRT		; GO CLOSE FOR WRITE

3288					; CLOSE FOR READ - FLAG CLOSED

3289	F030	A0 01					LDY	#SUCCES	; SUCCESSFULL

3290	F032	A9 3C			FCAX:		LDA	*MOTRST	; STOP THE MOTOR IN CASE WAS SHORT IRG MODE

3291	F034	8D 02	D3				STA	PACTL

3292	F037	60					RTS

3293	F038	A6 3D			CLWRT:	LDX	BPTR		; BUFFER POINTER

3294	F03A	F0 0A					BEQ	WTLR		; IF NO DATA BYTES IN BUFFER NO DTI REC

3295	F03C	8E 7F	04				STX	CASBUF+130	; WRITE TO LAST RECORD

3296	F03F	A9 FA					LDA	#DT1		; REC TYPE

3297	F041	20 D2	F0				JSR	WSIOSB	; WRITE OUT USER BUFFER

3298	F044	30 EC					BMI	FCAX		; GO IF ERROR

3299	F046	A2 7F			WTLR:		LDX	#127		; ZERO BUFFER

3300	F048	A9 00					LDA	#0

3301	F04A	9D 00	04		ZTBUF:	STA	CASBUF+3,X

3302	F04D	CA					DEX

3303	F04E	10 FA					BPL	ZTBUF

3304	F050	A9 FE					LDA	#EOT		; WRITE EOT RECORD

3305	F052	20 D2	F0				JSR	WSIOSB

3306	F055	4C 32	F0				JMP	FCAX		; FLAG CLOSED AND EXIT

3307						.PAGE

3308					;

3309					; SUBROUTINES

3310					;

3311					; BEEP - GENERATE TONE ON KEYBOARD SPEAKER

3312					; ON ENTRY A = FREQ

3313					;

3314	F058	85 40			BEEP:		STA	FREQ

3315	F05A	A5 14			BEEP1:	LDA	RTCLOK+2	; CURRENT CLOCK

3316	F05C	18					CLC

3317						.IF	PALFLG

3318							ADC	#25

3319						.ENDIF

3320						.IF	PALFLG-1

3321	F05D	69 1E					ADC	#30		; 1 SEC TONE

3322						.ENDIF

3323	F05F	AA					TAX

3324	F060	A9 FF			WFL:		LDA	#xFF

3325	F062	8D 1F	D0				STA	CONSOL	; TURN ON SPEAKER

3326	F065	A9 00					LDA	#0

3327	F067	A0 F0					LDY	#$F0

3328	F069	88					DEY

3329	F06A	D0 FD					BNE	#-1

3330	F06C	8D IF	D0				STA	CONSOL	; TURN OFF SPEAKER

3331	F06F	A0 F0					LDY	#$F0

3332	F071	88 					DEY

3333	F072	D0 FD					BNE	#-1

3334	F074	E414					CPX	RTCLOK+2	; SEE IF 1 SEC IS UP YET

3335	F076	D0 ES					BNE	WFL

3336	F078	C6 40					DEC	FREQ		; COUNT BEEPS

3337	F07A	F0 0B					BEQ	WFAK		; IF ALL DONE GO WAIT FOR KEY

3338	F07C	8A					TXA

3339	F07D	18					CLC

3340						.IF	PALFLG

3341							ADC	#8

3342						.ENDIF

3343						.IF	PALFLG-1

3344	F07E	69 0A					ADC	#10

3345						.ENDIF

3346	F080	AA					TAX

3347	F081	E4 14					CPX	RTCLOK+2

3348	F083	D0 FC					BNE	*-2

3349	F085	F0 D3					BEQ	BEEP1		; UNCOND GO BEEP AGIN

3350	F087	20 8C	F0		WFAK:		JSR	WFAK1		; USE SIMULATED "JMP (KGETCH)"

3351	F08A	98 					TYA

3352	F08B	60					RTS

3353	F08C	AD 25	E4		WFAK1:	LDA	KEYBDV+5

3354	F0SF	48					PHA

3355	F090	AD 24	E4				LDA	KEYBDV+4	; SIMULATE "JMP (KGETCH)"

3356	F093	48					PHA

3357	F094	60					RTS

3358					;

3359					; SIOSB - CALL SIO ON SYSTEM BUFFER

3360					;

3361	F095	8D 02	03		SIOSB:	STA	DCOMND	; SAVE COMMAND

3362	F098	A9 00					LDA	#0

3363	F09A	8D 09	03				STA	DBYTHI	; SET BUFFER LENGTH

3364	F09D	A9 83					LDA	*131

3365	F09F	8D 08	03				STA	DBYTLO

3366	F0A2	A9 03					LDA	#CBUFH

3367	F0A4	8D 05	03				STA	DBUFHI	; SET BUFFER ADDRESS

3368	F0A7	A9 FD					LDA	#CBUFL

3369	F0A9	8D 04	03				STA	DBUFLO

3370	F0AC	A9 60			CSIO:		LDA	#$60		; CASSET PSEUDO DEVICE

3371	F0AE	8D 00	03				STA	DDEVIC

3372	F0B1	A9 00					LDA	#0

3373	F0BS	8D 01	03				STA	DUNIT

3374	F0B6	A9 23					LDA	#35		; DEVICE TIMEOUT (5/30/79)

3375	F0BS	8D 06	03				STA	DTIMLO

3376	F0BS	AD 02	03				LDA	DCOMND	; GET COMMAND BACK

3377	F0BE	A0 40					LDY	#SRSTA	; SIO READ STATUS COMMAND

3378	F0C0	C9 52					CMP	#'R

3379	F0C2	F0 02					BEQ	*+4

3380	F0C4	A0 80					LDY	#SWSTA	; SIO WRITE STATUS COMMAND

3381	F0C6	8C 03	03				STY	DSTATS	; SET STATUS FOR SIO

3382	F0C9	A5 3E					LDA	FTYPE

3383	F0CB	8D 0B	03				STA	DAUX2		; INDICATE IF SHORT IRG MODE

3384	F0CE	20 59	E4				JSR	SIOV		; GO CALL SIO

3385	F0D1	60					RTS

3386					;

3387					; WSIOSB - WRITE SIO SYSTEM	BUFFER

3388					;

3389	F0D2	8D FF	03		WSIOSB:	STA	CASBUF+2	; STORE TYPE BYTE

3390	F0D5	A9 55					LDA	#455

3391	F0D7	8D FD	03				STA	CASBUF+0

3392	F0DA	8D FE	03				STA	CASBUF+1

3393	F0DD	A9 57					LDA	#'W 		; WRITE

3394	F0DF	20 95	F0				JSR	SIOSB		; CALL SIO ON SYSTEM BUFFER

3395	F0E2	60					RTS	AND	; RETURN

3396	F0E3				CRNTP6	=*

3397							*=$14

3398	0014	00			CASSPR:	.BYTE	MONORG-CRNTP6 ; ^GCASCV IS TOO LONG

3399					;