3400					.TITLE	'MONITOR ****# MONITP.SRC ***** 3/9/79 ***** 4:00:00 P

3401					;

3402					;

3403					;

3404					; 	CONSTANT EQUATES

3405					;

3406	0009				PUTTXT	=	$9		; "PUT TEXT RECORD" CIO COMMAND CODE

3407	0007				GETCAR	=	$7		; "GET CHARACTER" CIO COMMAND CODE

3408	000B				PUTCAR	=	$B		; "PUT CHARACTER" CIO COMMAND CODE

3409	0000				INIMLL	=	$00		; INITIAL MEM LO LOW BYTE

3410	0007				INIMLH	=	$07		; INITIAL MEM LO HIGH BYTE

3411					; GOOD	=	$1		; GOOD STATUS CODE

3412					; WRITE	=	$57		; WRITE COMMAND

3413					; READ	=	$52		; READ COMMAND

3414					; STATC	=	$53		; STATUS COMMAND

3415	0000				SEX		=	$0		; SCREEN EDITOR IOCB INDEX

3416	007D				CLS		=	$71)		; CLEAR SCREEN CODE

3417	0092				CTRLC		=	$92		; KEYBOARD CODE FOR 'CONTROL C'

3418	0088				EOF		=	388		; CASSETTE END OF FILE CODE

3419	0000				LIRG		=	$0		; LONG IRG TYPE CODE

3420					;

3421	0004				BUFFH		=	(CASBUF+3)/256

3422	0000				BUFFL		=	(-256)*BUFFH+CASBUF+3 ; BUFFER POINTER

3423					;

3424					;

3425					;

3426					; THE FOLLOWING EQUATES ARE IN THE CARTRIDGE ADDRESS SPACE.

3427					;

3428					;

3429					; "B" CARTRIDGE	ADDR'S	ARE 8000-9FFF (36K CONFIG. ONLY)

3430					; "A" CART. ADDR'S ARE	A000-BFFF (36K CONFIG. ONLY)

3431					;

3432					; "A" CART. ADDR'S ARE	B000-BFFF (48K CONFIG. ONLY)

3433					;

3434							*=$BFFA

3435	BFFA				CARTCS:	.RES	2		; CARTRIDGE COLD START ADDRESS.

3436	BFFC				CART:		.RES	1		; CARTRIDGE AVAILABLE FLAG BYTE.

3437	BFFD				CARTFG:	.RES	1		; CARTRIDGE FLAG BYTE. BIT 0=FLAG1~

3438	BFFE				CARTAD:	.RES	2		; 2-BYTE CARTRIDGE START VECTOR

3439					;

3440					;

3441					; CARTRIDGE FLAG	ACTION DEFINITIONS

3442					;

3443					;

3444					; 	BIT		ACTION IF SET

3445					;

3446					; 	7		SPECIAL -- DON'T POWER-UP, JUST RUN CARTRIDGE

3447					; 	6-3		NONE

3448					; 	2		RUN CARTRIDGE

3449					; 	1		NONE

3450					; 	0		BOOT DOS

3451					; 	3452

3453					; 	*****

3454					; 	NOTE

3455					; 	*****

3456					;

3457					; 	1. IF BIT2 IS 0. GOTO BLACKBOARD	MODE.

3458					;                2. IF BITO SET. THE	DISK WILL BE	BOOTED BEFORE ANY

3459					; OTHER ACTION.

3460					;

3461					;

3462					;

3463					;

3464					;

3465					;

3466					;

3467					;

3468					;

3469					; POWER-UP VECTOR

3470					;

3471					; ****************************

3472					*=$FFFC

3473					;

3474					; PVECT .WORD PWRUP  	POWER-UP VECTOR

3475					; *************************

3476					;

3477					;

3478					;

3479					;

3480					;

3481					; 	ENTRY POINT VECTOR

3482					;

3483							*=BLKBDV

3484					;

3485	E471	4C 23	F2				JMP 	SIGNON		; BLACK BOARD VECTOR

3486					;

3487					*=WARMSV

3488					;

3489	E474	4C 1B	F1	   			JMP 	RESET	:WARM START	VECTOR

3490					;

3491					*=COLDSV

3492					;

3493	E477	4C 25	F1				JMP 	PWRUP		; COLD START VECTOR (9000 FOR RAM VECTOR WRIT

3494					;

3495							*=$9000

3496	9000	20 0C	90				JSR 	$900C

3497	9003	4C 25	F1				JMP 	PWRUP		; (TO HANDLE	RAM VECTOR WRITING)

3498	9006	20 0C	90				JSR 	#900C

3499	9009	4C 18	Fl				JMP 	RESET

3500					;

3501					;

3502					;

3503							*=MONORG

3504					;

3505					;

3506					;

3507					;

3508					; 	HANDLER TABLE ENTRIES

3509					;

3510	F0ES	50			TBLENT:	.BYTE 'P'

3511	F0E4	30 E4					.WORD PRINTV

3512	F0E6	43					.BYTE 'C'

3513	F0E7	40 E4					.WORD CASETV

3514	F0ES	45 					.BYTE 'E'

3515	F0EA	00 E4					.WORD EDITRV

3516	F0EC	53					.BYTE 'S'

3517	F0ED	10 E4					.WORD SCRENV

3518	F0EF	4B					.BYTE 'K'

3519	F0F0	20 E4					.WORD KEYBDV

3520					;

3521					;

3522					; TBLLEN = IDENT-TBLENT-1 HANDLER TABLE LENGTH. "MOVED TO LINE 8

3523					,

3524					; 	***** PRINT MESSAGES *****

3525					;

3526					;

3527	F0F2	7D 41	54 41		IDENT:	.BYTE CLS,'ATARI COMPUTER - MEMO PAD ',CR

3528	F0F6	52 49 20 43

3529	F0FA	4F 4D	50 55

3530	F0FE	54 45	52 20

3531	F102	2D 20	4D 45

3532	F106	4D 4F	20 50

3533	F10A	41 44	9B

3534					;

3535	00F0				IDENTH	= 	IDENT/256

3536	00F2				IDENTL	= 	(-256)*IDENTH+IDENT 	; SYSTEM I. D. MSG POINTER

3537					;

3538	000E					TBLLEN	= IDENT-TBLENT-1 ; HANDLER TABLE LENGTH

3539	F10D	42 4F	4F 54		DERR5:	.BYTE 'BOOT ERROR ',CR

3540	F111	20 45	52 52

3541	F115	4F 52	9B

3542

3543	00F1				DERRH		=	DERR5/256

3544	000D				DERRL		=	(-256)*DERRH+DERR5 	; DISK ERROR MSG POINTER

3545					;

3546					;

3547					;

3548					;

3549					; 	DEVICE/FILENAME SPECIFICATIONS

3550					;

3551	F118	45 3A	9B		OPNEDT:	.BYTE 'E: ',CR ; "OPEN SCREEN EDITOR" DEVICE SPEC.

3552					;

3553	00F1				OPNH		= 	OPNEDT/256

3554	0018				OPNL		=	(-256)*OPNH+OPNEDT 	; SCREEN EDITOR OPEN POINTER

3555	F11B				;

3556					;

3557					;

3558					;

3559					;

3560					; *****************************************************************

3561					; 			RESET BUTTON ROUTINE STARTS HERE

3562					; *****************************************************************

3563

3564	F11B	78			RESET: SEI	; DISABLE IRG INTERRUPTS

3565	F11C	AD 44	02				LDA	COLDST 	; WERE WE IN MIDDLE OF COLDSTART?

3566	F11F	D0 04					BNE	PWRUP 	; YES. GO TRY IT AGAIN

3567	F121	A9 FF					LDA	#$FF

3568	F123	D0 03					BNE	PWRUPI 	; SET WARM START FLAG

3569					;

3570					;

3571					;

3572					; *****************************************************************

3573					;			POWER	UP ROUTINES START HERE

3574					; *****************************************************************

3575					;

3576	F125	78			PWRUP: 	SEI			; DISABLE IRQ INTERRUPTS

3577	F126	A9 00					LDA	#0 		; CLEAR WARMSTART FLAG

3578	F128	85 08			PWRUPI: 	STA	WARMST

3579	F12A	D8					CLD			; CLEAR DECIMAL FLAG.

3580	F12B	A2 FF					LDX	#$FF

3581	F12D	9A					TXS			; SET STACK POINTER

3582	F12E	20 3F	F2				JSR	SPECL 	; CARTRIDGE SPECIAL CASE?

3583	F131	20 77	F2				JSR	HARDI 	; DO HARDWARE INITIALIZATION

3584	F134	A5 08					LDA	WARMST 	; IS IT WARMSTART?

3585	F136	D0 28					BNE	ZOSRAM 	; YES, ONLY ZERO OS RAM

3586					;

3587	F138	A9 00			ZERORM: LDA	1#0

3588	F13A	A0 08					LDY	#WARMST

3589	F13C	85 04					STA	RAMLO

3590	F13E	85 05					STA	RAMLO+1 	; INITIALIZE RAM POINTER

3591	F140	91 04			CLRRAM: STA	(RAMLO),Y 		; CLEAR MEMORY LOC.

3592	F142	C8					INY

3593	F143	C0 00					CPY	#10 		; AT END OF PAGE?

3594	F145	D0 F9					BNE	CLRRAM

3595	F147	E6 05					INC	RAML0+1	; YES. INCR PAGE POINTER

3596	F149	A6 05					LDX	RAMLO+1

3597	F14B	E4 06					CPX	TRAMSZ 	; AT END OF MEM?

3598	F14D	D0 F1					BNE	CLRRAM 	; NO.

3599					;

3600					; INITIALIZE DOSVEC TO POINT TO SIGNON (BLACKBOARD)

3601	F14F	AD 72	E4				LDA	BLKBDV+i

3602	F152	85 0A					STA	DOSVEC 	; USE BLACKBOARD VECTOR

3603	F154	AD 73	E4				LDA	BLKBDV+2 	; FOR DOSVEC

3604	F157	85 0B					STA	DOSVEC+1

3605	F159	A9 FF					LDA	#$FF

3606	F15B	8D 44	02				STA	COLDST 	; SET TO SHOW IN MIDDLE OF COLDSTART

3607	F15E	D0 13					BNE	ESTSCM 	; GO AROUND ZOSRAM

3608					;

3609					; CLEAR OS RAM	(FOR WARMSTART)

3610	F160	A2 00			ZOSRAM: 	LDX	#0

3611	F162	8A					TXA

3612	F163	9D 00	02		ZOSRM2: 	STA	#200,X 	; CLEAR PAGES 2 AND 3

3613	F166	9D 00	03				STA	$300,X

3614	F169	CA					DEX

3615	F16A	D0 F7					BNE	ZOSRM2

3616	F16C	A2 10					LDX	#INTZBS

3617	F16E	95 00			ZOSRM3:	STA	0,X		; CLEAR ZERO PAGE LOCATIONS INTZBS-7F

3618	F170	E8					INX

3619	F171	10 FB					BPL	ZOSRM3

3620					;

3621					; ESTABLISH SCREEN MARGINS

3622	F173	A9 02			ESTSCM:	LDA	#LEDGE

3623	F175	85 52					STA	LMARGN

3624	F177	A9 27					LDA	#REDGE

3625	F179	85 53					STA	RMARGN

3626					;

3627					;

3628					; MOVE VECTOR TABLE FROM ROM TO RAM

3629	F17B	A2 25			OPSYS:	LDX	#$25

3630	F17D	BD 80	E4		MOVVEC:	LDA	VCTABL,X	;ROM TABLE

3631	F180	9D 00	02				STA	INTABS,X	; TO RAM

3632	F183	CA					DEX

3633	F184	10 F7					BPL	MOVVEC

3634	F186	20 8A	F2				JSR	OSRAM		; DO O. S. RAM SETUP

3635	F189	58					CLI			; ENABLE IRG INTERRUPTS

3636					;

3637					;

3638					;	LINK HANDLERS

3639					;

3640	F18A	A2 0E					LDX	#TBLLEN

3641	F18C	BD E3	F0		NXTENT:	LDA	TBLENT,X	; READ HANDLER TABLE ENTRY

3642	F18F	9D 1A	03				STA	HATABS,X	; PUT IN TABLE

3643	F192	CA					DEX

3644	F193	10 F7					BPL	NXTENT	; DONE WITH ALL ENTRIES?

3645					;

3646					;

3647					;

3648					;

3649					;

3650					; INTERROGATE	CARTRIDGE ADDR. SPACE TO SEE WHICH CARTRIDGES THERE ARE

3651					;

3652	F195	A2 00					LDX	#0

3653	F197	86 07					STX	TSTDAT	; CLEAR "B" CART. FLAG

3654	F199	86 06					STX	TRAMSZ	; CLEAR "A" CART. FLAG

3655	F19B	AE E4	02				LDX	RAMSIZ

3656	F19E	E0 90					CPX	#$90		; RAM IN "B" CART. SLOT?

3657	F1A0	B0 0A					BCS	ENDBCK

3658	F1A2	AD FC	9F				LDA	CART-$2000	; NO,

3659	F1A5	D0 05					BNE	ENDBCK	; CART. PLUGGED INTO "B" SLOT?

3660	F1A7	E6 07					INC	TSTDAT	; YES, SET "B" CART. FLAG

3661	F1A9	20 3C	F2				JSR	CBINI		; INITIALIZE CARTRIDGE "B"

3662					;

3663	F1AC	AE E4	02		ENDBCK:	LDX	RAMSIZ

3664	F1AF	E0 B0					CPX	#$B0		; RAM IN "A" CART. SLOT?

3665	F1B1	B0 0A					BCS	ENDACK

3666	F1B3	AE FC	BF				LDX	CART		; NO,

3667	F1B6	D0 05					BNE	ENDACK	; CART. PLUGGED INTO "A" SLOT?

3668	F1B8	E6 06					INC	TRAMSZ	; YES, SET "A" CART. FLAG

3669	F1BA	20 39	F2				JSR	CAINI		; INITIALIZE CARTRIDGE "A"

3670					;

3671					;

3672					; OPEN	SCREEN EDITOR

3673

3674	F1BD	A9 03			ENDACK:	LDA	113

3675	F1BF	A2 00					LDX	#SEX

3676	F1C1	9D 42	03				STA	ICCOM,X	; OPEN I/0 COMMAND

3677	F1C4	A9 18					LDA	#OPNL

3678	F1C6	9D 44	03				STA	ICBAL,X

3679	F1C9	A9 F1					LDA	#0PNH

3680	F1CB	9D 45	03				STA	ICBAH,X	; SET BUFFER POINTER TO OPEN SCREEN EDITOR

3681	F1CE	A9 0C					LDA	#$C

3682	F1D0	9D 4A	03				STA	ICAX1,X	; SET UP OPEN FOR INPUT/OUTPUT

3683	F1D3	20 56	E4				JSR	CIOV		; GO TO CIO

3684					;

3685	F1D6	10 03					BPL	SCRNOK	; BR IF NO ERROR

3686	F1D8	4C 25	F1				JMP	PWRUP		; RETRY PWRUP IF ERROR (SHOULD NEVER HAPPEN!)

3687	F1DB	E8			SCRNOK:	INX			; SCREEN OK, SO WAIT FOR VBLANK TO

3688	F1DC	D0 FD					BNE	SCRNOK	; BRING UP THE DISPLAY

3689	F1DE	C8					INY

3690	F1DF	10 FA					BPL	SCRNOK

3691					;

3692					;

3693					; 		DO CASSETTE	BOOT

3694	F1E1	20 B2	F3				JSR	CSBOOT	; CHECK, BOOT, AND INIT

3695					;

3696					; CHECK TO SEE IF EITHER CARTRIDGE WANTS DISK BOOT

3697	F1E4	A5 06					LDA	TRAMSZ	; CHECK BOTH CARTRIDGES

3698	F1E6	05 07					ORA	TSTDAT

3699	F1EB	F0 12					BEQ	NOCART	; NEITHER CARTRIDGE LIVES

3700	F1EA	A5 06					LDA	TRAMSZ	; "A" CART?

3701	F1EC	F0 03					BEQ	NOA1		; NO

3702	F1EE	AD FD	BF				LDA	CARTFG	; GET CARTRIDGE MODE FLAG

3703	F1F1	A6 07			NOA1:		LDX	TSTDAT	; "B" CART?

3704	F1F3	F0 03					BEQ	NOB1		; NO

3705	F1F5	0D FD	9F				ORA	CARTFG-$2000 	; ADD OTHER FLAG

3706	F1FE	29 01			NOB1:		AND	#1		; DOES EITHER CART WANT BOOT?

3707	F1FA	F0 03					BEQ	NOBOOT	; NO

3708					;

3709					; 		DO DISK BOOT

3710	F1FC	20 CF	F2		NOCART:	JSR	BOOT		; CHECK, BOOT, AND INIT

3711					;

3712					; GO TO	ONE OF THE CARTRIDGES IF THEY SO DESIRE

3713	F1FF	A9 00			NOBOOT:	LDA	#O

3714	F201	8D 44	02				STA	COLDST	; RESET TO SHOW DONE WITH COLDSTART

3715	F204	A5 06					LDA	TRAMSZ	; "A" CART?

3716	F206	F0 0A					BEQ	NOA2		; NO

3717	F208	AD FD	BF				LDA	CARTFG	; GET CARTRIDGE MODE FLAG

3718	F20B	29 04					AND	#4		; DOES IT WANT TO RUN?

3719	F20D	F0 03					BEQ	NOA2		; NO

3720	F20F	6C FA	BF				JMP	(CARTCS)	; RUN "A" CARTRIDGE

3721	F212	A5 07			NOA2:		LDA	TSTDAT	; "B" CART?

3722	F214	F0 0A					BEQ	NOCAR2	; NO

3723	F216	AD FD	9F				LDA	CARTFG-$2000 	; GET "B" MODE FLAG

3724	F219	29 04					AND	#4 		; DOES IT WANT TO RUN'?

3725	F21B	F0 DF					BEQ	NOCART 	; NO

3726	F21D	6C FA	9F				JMP	(CARTCS-$2000) ; RUN "B" CARTRIDGE

3727		 			;

3728					; NO CARTRIDGES	OR NEITHER WANTS TO RUN,

3729					; SO GO TO DOSVEC (DOS, CASSETTE, OR BLACKBOARD)

3730	F220	6C 0A	00		NOCAR2:	JMP	(DOSVEC)

3731					;

3732					; PRINT	SIGN-ON	MESSAGE

3733	F223	A2 F2			SIGNON:	LDX	#IDENTL

3734	F225	A0 F0					LDY	#IDENTH

3735	F227	20 85	F3				JSR	PUTLIN 	; GO PUT SIGN-ON MSG ON SCREEN

3736					;

3737					;

3738					;

3739					; 	BLACKBOARD ROUTINE

3740	F22A	20 30	F2		BLACKS:	JSR	BLKB2 	; "JSR EGETCH"

3741	F22D	4C 2A	F2				JMP	BLACKS 	; FOREVER

3742	F230	AD 05	E4		BLKB2:	LDA	EDITRV+5 	; HIGH BYTE

3743	F233	48					PHA

3744	F234	AD 04	E4				LDA	EDITRV+4 	; LOW BYTE

3745	F237	48					PHA

3746	F238	60					RTS			; SIMULATES "JMP (EDITRV)"

3747					;

3748					;

3749					; CARTRIDGE INITIALIZATION INDIRECT JUMPS

3750	F239	6C FE	BF		CAINI:	JMP	(CARTAD)

3751	F23C	6C FE	9F		CBINI:	JMP	(CARTAD-$2000)

3752						.PAGE

3753					;

3754					;

3755					;

3756					;

3757					;

3758					;

3759					;		S U B R O U T I N E S

3760					;

3761					;

3762					;

3763					;

3764					;

3765					;

3766					;

3767					;

3768					;

3769					;

3770					;

3771					;

3772					;

3773					;

3774					;

3775					;

3776					;

3777					;

3778					;	CHECK	FOR HOW	MUCH RAM &	SPECIAL CARTRIDGE CASE.

3779					;	IF SPECIAL CARTRIDGE CASE. DON'T GO BACK -- GO TO CART.

3780					;

3781	F23F	AD FC	BF		SPECL:	LDA	CART		; CHECK FOR RAM OR CART

3782	F242	D0 13					BNE	ENSPE2	; GO IF NOTHING OR MAYBE RAM

3783	F244	EE FC	BF				INC	CART		; NOW DO RAM CHECK

3784	F247	AD FC	BF				LDA	CART		; IS IT ROM?

3785	F24A	D0 08					BNE	ENSPEC	; N0

3786	F24C	AD FD	BF				LDA	CARTFG	; YES,

3787	F24F	10 03					BPL	ENSPEC	; BIT SET?

3788	F251	6C FE	BF				JMP	(CARTAD)	; YES. GO RUN CARTRIDGE

3789

3790					; CHECK	FOR AMOUNT OF RAM

3791					;

3792					;

3793	F254	CE FC	BF		ENSPEC:	DEC	CART		; RESTORE RAM IF NEEDED

3794	F257	A0 00			ENSPE2:	LDY	#0

3795	F259	84 05					STY	RAMLO+1

3796	F25B	A9 10					LDA	#$10

3797	F25D	85 06					STA	TRAMSZ	; SET RAM POINTER TO 4K.

3798	F25F	B1 05			HOWMCH:	LDA	(RAMLO+1),Y	; READ RAM LOCATION

3799	F261	49 FF					FOR	#$FF		; INVERT IT.

3800	F263	91 05					STA	(RAMLO+1),Y	; WRITE INVERTED DATA.

3801	F265	D1 05					CMP	(RAMLO+1),Y	; READ RAM AGAIN

3802	F267	D0 0D					BNE	ENDRAM

3803	F269	49 FF					EOR	#$FF		; CONVERT IT BACK

3804	F26B	91 05					STA	(RAMLO+1),Y	; RESTORE ORIGINAL RAM DATA

3805	F26D	A5 06					LDA	TRAMSZ

3806	F26F	18					CLC

3807	F270	69 10					ADC	#$10

3808	F272	85 06					STA	TRAMSZ	; INCR. RAM POINTER BY 4K.

3809	F274	D0 E9					BNE	HOWMCH	; GO FIND HOW MUCH RAM.

3810	F276	60			ENDRAM:	RTS

3811					;

3812					;

3813					;

3814					;

3815					; 	HARDWARE INITIALIZATION

3816					;

3817					;

3818	F277	A9 00			HARDI:	LDA	#0

3819	F279	AA					TAX

3820	F27A	9D 00	D0		CLRCHP:	STA	$D000,X

3821	F27D	9D 00	D4				STA	$D400,X

3822	F280	9D 00	D2				STA	$D200,X

3823	F283	9D 00	D3				STA	$D300,X

3824	F286	E8					INX

3825	F287	D0 F1					BNE	CLRCHP

3826	F289	60					RTS

3827					;

3828					;

3829					; 	O. S.	RAM SETUP

3830					;

3831	F28A	C6 11			OSRAM:	DEC	BRKKEY	; TURN OFF BREAK KEY FLAG

3832	F28C	A9 54					LDA	#.LOW.BRKKY2

3833	F28E	8D 36	02				STA	BRKKY

3834	F291	A9 E7					LDA	#.HIGH.BRKKY2

3835	F293	8D 37	02				STA	BRKKY+1

3836	F296	A5 06					LDA	TRAMSZ	; READ RAM SIZE IN TEMP. REG.

3837	F298	8D E4	02				STA	RAMSIZ	; SAVE IT IN RAM SIZE.

3838	F29B	8D E6	02				STA	MEMTOP+1	; INIT. MEMTOP ADDR HI BYTE

3839	F29E	A9 00					LDA	#0

3840	F2A0	8D E5	02				STA	MEMTOP	; INIT. MEMTOP ADDR LO BYTE

3841	F2A3	A9 00					LDA	#INIMLL

3842	F2A5	8D E7	02				STA	MEMLO

3843	F2AS	A9 07					LDA	#INIMLH

3844	F2AA	8D E8	02				STA	MEMLO+1	; INITIALIZE MEMLO ADDR VECTOR

3845	F2AD	20 0C	E4				JSR	EDITRV+$C	; EDITOR INIT.

3846	F2B0	20 1C	E4				JSR	SCRENV+$C	; SCREEN INIT.

3847	F2B3	20 2C	E4				JSR	KEYBDV+$C	; KEYBOARD INIT.

3848	F2B6	20 3C	E4				JSR	PRINTV+$C	; PRINTER HANDLER INIT

3849	F2B9	20 4C	E4				JSR	CASETV+$C	; CASSETTE HANDLER INIT

3850	F2BC	20 6E	E4				JSR	CIOINV	; CIO INIT.

3851	F2BF	20 65	E4				JSR	SIOINV	; SIO INIT.

3852	F2C2	20 68	E4				JSR	INTINV	; INTERRUPT HANDLER INIT.

3853	F2C5	AD 1F	D0				LDA	CONSOL

3854	F2C8	29 01					AND	#$1

3855	F2CA	D0 02					BNE	NOKEY		; GAME START KEY DEPRESSED?

3856	F2CC	E6 4A					INC	CKEY		; YES, SET KEY FLAG.

3857	F2CE	60 			NOKEY:	RTS

3858					;

3859					;

3860					; DO BOOT OF DISK

3861					;

3862	F2CF	A5 08			BOOT:		LDA	WARMST

3863	F2Dl	F0 0A					BEQ	NOWARM	; WARM START?

3864	F2D3	AS 09					LDA	BOOT?		; YES

3865	F2D5	29 01					AND	#1

3866	F2D7	F0 03					BEQ	NOINIT	; VALID BOOT?

3867	F2D9	20 7E	F3				JSR	DINI		; YES, RE-INIT DOS SOFTWARE

3868	F2DC	60			NOINIT:	RTS

3869	F2DD	A9 01			NOWARM:	LDA	#1

3870	F2DF	8D 01	03				STA	DUNIT		; ASSIGN DISK DRIVE NO.

3871	F2E2	A9 53					LDA	#STATC

3872	F2E4	8D 02	03				STA	DCOMND	; SET UP STATUS COMMAND

3873	F2E7	20 53	E4				JSR	DSKINV	; GO DO DISK STATUS

3874	F2EA	10 01					BPL	DOBOOT	; IS STATUS FROM SIO GOOD?

3875	F2EC	60					RTS			; NO. GO BACK WITH BAD BOOT STATUS

3876					;

3877	F2ED	A9 00			DOBOOT:	LDA	#0

3878	F2EF	8D 0B	03				STA	DAUX2

3879	F2F2	A9 01					LDA	#1

3880	F2F4	8D 0A	03				STA	DAUX1		; SET SECTOR # TO 1.

3881	F2F7	A9 00					LDA	#BUFFL

3882	F2F9	8D 04	03				STA	DBUFLO

3883	F2FC	A9 04					LDA	#BUFFH

3884	F2FE	8D 05	03				STA	DBUFHI	; SET UP BUFFER ADDR

3885	F301	20 9D	F3		SECT1:	JSR	GETSEC	; GET SECTOR

3886	F304	10 08					BPL	ALLSEC	; STATUS O. K. ?

3887	F306	20 81	F3		BADDSK:	JSR	DSKRDE	; NO. GO PRINT DISK READ ERROR

3888	F309	A5 4B					LDA	CASSBT

3889	F30B	F0 E0					BEQ	DOBOOT	; CASSETTE BOOT?

3890	F30D	60					RTS			; YES. QUIT

3891	F30E	A2 03			ALLSEC:	LDX	#3

3892	F310	BD 00	04		RDBYTE:	LDA	CASBUF+3,X	; READ A BUFFER BYTE

3893	F313	9D 40	02				STA	DFLAGS,X	; STORE IT

3894	F316	CA					DEX

3895	F317	10 F7					BPL	RDBYTE	; DONE WITH 4 BYTE TRANSFER ?

3896	F319	AD 42	02				LDA	BOOTAD	; YES.

3897	F31C	85 04					STA	RAMLO

3898	F31E	AD 43	02				LDA	BOOTAD+i

3899	F321	85 05					STA	RAMLO+1	;PUT BOOT ADDR INTO Z. PAGE RAM

3900	F323	AD 04	04				LDA	CASBUF+7

3901	F326	85 0C					STA	DOSINI	; ESTABLISH DOS INIT ADDRESS

3902	F328	AD 05	04				LDA	CASBUF+B

3903	F32B	85 0D					STA	DOSINI+1

3904	F32D	A0 7F			MVBUFF:	LDY	#$7F		; YES, SET BYTE COUNT

3905	F32F	B9 00	04		MVNXB:	LDA	CASBUF+3,Y

3906	F332	91 04					STA	(RAMLO),Y	; MOVE A BYTE FROM SECTOR BUFFER TO BOOT ADDR

3907	F334	88					DEY

3908	F335	10 F8					BPL	MVNXB		; DONE ?

3909	F337	18					CLC			; YES,

3910	F338	A5 04					LDA	RAMLO

3911	F33A	69 80					ADC	#$80

3912	F33C	85 04					STA	RAMLO

3913	F33E	A5 05					LDA	RAMLO+1

3914	F340	69 00					ADC	#0

3915	F342	85 05					STA	RAMLO+1	; INCR BOOT LOADER BUFFER POINTER.

3916	F344	CE 41	02				DEC	DBSECT	; DECR # OF SECTORS.

3917	F347	F0 11					BEQ	ENBOOT	; MORE SECTORS ?

3918	F349	EE 0A	03				INC	DAUX1		; YES. INCR SECTOR #

3919	F34C	20 9D	F3		SECTX: 	JSR	GETSEC	; GO GET SECTOR.

3920	F34F	10 DC					BPL	MVBUFF	; STATUS 0. K. ?

3921	F351	20 81	F3				JSR	DSKRDE	; NO. GO PRINT DISK READ ERROR

3922	F354	A5 4B					LDA	CASSBT

3923	F356	D0 AE					BNE	BADDSK	; IF CASSETTE QUIT.

3924	F358	F0 F2					BEQ	SECTX		; IF DISK TRY SECTOR AGAIN.

3925	F35A	A5 4B			ENBOOT: 	LDA	CASSBT

3926	F35C	F0 03					BEQ	XBOOT		; CASSETTE BOOT ?

3927	F35E	20 9D	F3				JSR	GETSEC	; YES, GET EOF RECORD BUT DON'T USE IT.

3928	F361	20 6C	F3		XBOOT: 	JSR	BLOAD		; GO EXECU"~E BOOT LOADER

3929	F364	B0 A0					BCS	BADDSK	; IF BAD BOOT. DO IT OVER AGAIN

3930	F366	20 7E	F3				JSR	DINI		; GO INIT. SOFTWARE

3931	F369	E6 09					INC	BOOT?		; SHOW BOOT SUCCESS

3932	F36B	60					RTS

3933	F36C	18			BLOAD: 	CLC

3934	F36D	AD 42	02				LDA	BOOTAD

3935	F370	69 06					ADC	#6

3936	F372	85 04					STA	RAMLO

3937	F374	AD 43	02				LDA	BOOTAD+1

3938	F377	69 00					ADC	#0

3939	F379	85 05					STA	RAMLO+1	;PUT START ADDR OF BOOTLOADER INTO RAM

3940	F37B	6C 04	00				JMP	(RAMLO)

3941	F37E	6C 0C	00		DINI: 	JMP	(DOSINI)

3942

3943

3944

3945

3946					; DISPLAY DISK	READ ERROR MSG

3947

3948	F381	A2 0D			DSKRDE: LDX	#DERRL

3949	F383	A0 F1					LDY	#DERRH

3950					;

3951					;

3952					;

3933					; PUT LINE ON SCREEN AT PRESENT CURSOR POSITION

3954					;

3955					; X-REG -- LO	BYTE BEGIN	ADDR OF LINE

3956					; Y-REG -- HI	BYTE BEGIN	ADDR OF LINE

3957					;

3958	F385	8A			PUTLIN: 	TXA

3959	F386	A2 00					LDX	#SEX

3960	F388	9D 44	03				STA	ICBAL,X

3961	F38B	98 					TYA

3962	F38C	9D 45	03				STA	ICBAH,X	; SET UP ADDR OF BEGIN OF LINE

3963	F38F	A9 09					LDA	#PUTTXT

3964	F391	9D 42	03				STA	ICCOM,X	; "PUT TEXT RECORD" COMMAND

3965	F394	A9 FF					LDA	#; FF

3966	F396	9D 48	03				STA	ICBLL,X	; SET BUFFER LENGTH

3967	F399	20 56	E4				JSR	CIOV		;PUT LINE ON SCREEN

3968	F39C	60					RTS

3969					;

3970					;

3971					;

3972					;

3973					; GET SECTOR FROM DISK 0

3974					;

3975	F39D	A5 4B			GETSEC:	LDA	CASSBT

3976	F39F	F0 03					BEQ	DISKM		; CASSETTE BOOT ?

3977	F3A1	4C 7A	E4				JMP	RBLOKV	; YES GO TO READ BLOCK ROUTINE

3978	F3A4	A9 52			DISKM:	LDA	#READ

3979	F3A6	8D 02	03				STA	DCOMND	; SET READ SECTOR COMMAND

3980	F3A9	A9 01					LDA	#1

3981	F3AB	8D 01	03				STA	DUNIT		; SET DRIVE NO. TO DRIVE 0

3982	F3AE	20 53	E4				JSR	DSKINV	; GET SECTOR

3983	F3B1	60					RTS

3984

3985

3986					;

3987					; DO CHECK FOR	CASSETTE BOOT & fF SO. DO BOOT

3988					;

3989	F3B2	A5 08			CSBOOT:	LDA	WARMST	; WARMSTART?

3990	F3B4	F0 0A					BEQ	CSBOT2	; NO

3991	F3B6	A5 09					LDA	BOOT?		; GET BOOT FLAG

3992	F388	29 02					AND	#2		; WAS CASSETTE BOOT SUCCESFULL?

3993	F3BA	F0 03					BEQ	NOCSB2	; NO

3994	F3BC	20 E1	F3				JSR	CINI		; YES INIT CASSETTE SOFTWARE

3995	F3BF	60			NOCSB2:	RTS

3996					;

3997	F3C0	A5 4A			CSBOT2:	LDA	CKEY

3998	F3C2	F0 1C					BEQ	NOCSBT	; "C" KEY FLAG SET ?

3999	F3C4	A9 80					LDA	#$80		; YES.

4000	F3C6	85 3E					STA	FTYPE		; SET LONG IRG TYPE

4001	F3C8	E6 48					INC	CASSBT	; SET CASSETTE BOOT FLAG

4002	F3CA	20 7D	E4				JSR	CSOPIV	; OPEN CASSETTE FOR INPUT

4003	F3CD	20 01	F3				JSR	SECT1		; DO BOOT & INIT.

4004	F3D0	A9 00					LDA	#0

4005	F3D2	85 4B					STA	CASSBT	;RESET CASSETTE BOOT FLAG

4006	F3D4	85 4A					STA	CKEY		; CLEAR KEY FLAG

4007	F3D6	06 09					ASL	BOOT?		; SHIFT BOOT FLAG (NOW=2 IF SUCCESS)

4008	F3D8	A5 0C					LDA	DOSINI

4009	F3DA	85 02					STA	CASINI	; MOVE INIT ADDRESS FOR CASSETTE

4010	F3DC	A5 0D					LDA	DOSINI+1

4011	F3DE	85 03					STA	CASINI+1

4012	F3E0	60 			NOCSBT:	RTS

4013

4014	F3E1	6C 02	00		CINI:		JMP	(CASINI)	; INIT CASSETTE

4015					; ***********************************************************************

4016					;

4017					;

4018					; SPARE	BYTE OR	MODULE TOO	LONG FLAG

4019					;

4020	F3E4				CRNTP7	=*

4021					;

4022 							*=$14

4023 0014 00 			MONSPR: .BYTE KBDORG-CRNTP7 :^GMONITP TOO LONG